# MICROVITEC

## SERVICE MANUAL

series — 3
colour display monitors

GP0019AA2

MICROVITEC SERVICE MANUAL

INTRODUCTION

The contents of this manual are divided into several sections.

We will now look in some detail at the sections and explain how they are to be used to their best advantage.

Factory Pre-set Adjustments:

Some of these adjustments are very critical, not only from an operational point but also from a safety point. The manufacturer recommends that you read and understand the section fully before making any adjustments.

All pre-set adjustments are included in this section. You are given the prefix for example: Field Linearity VR312. If you are not sure of the location of any pre-set (or component) then you should refer to the section containing PCB or CRT base panel layouts. Once you have located the pre-set, you are then told what effect the adjustment will have on the display.

Removal/Installation (Mechanical Detail):

This section gives information (text and illustrated) on how to remove major service items such as: CRT, Main PCB and Tripler etc.

Description and Operation:

These sections will help you to understand the design concept of the Microvitec 'CUB' Monitor. They give very detailed descriptions of each part of the circuit. For example: Line Timebase. Field Timebase etc. It is very important that you read these sections before attempting any of the fault isolation and repair procedures.

Specification:

The first specification list (1431/1432) contains a full specification; subsequent lists contain only the areas that differ from the first list. For example: the supply is 180-265 volts, 48-64 Hz (excluding 110 volt models). This will only be given on the first specification list. Therefore, unless otherwise stated, refer to specification 1.

Circuit Diagram and Parts Listing:

There are many model variants in the Microvitec 'CUB' range and obviously it would not be practical to supply all the circuit diagrams. We have therefore devised one circuit diagram for the main P.C.B. assembly (including Standard/Medium Resolution Tube Base assembly) and a separate circuit diagram for the High Resolution Tube Base assembly. In addition there are no component values on the circuit diagrams: you should refer to the Parts Listing.

Component location is another problem that you may encounter. To assist you in this matter we have also included a full P.C.B. layout.

Fault Isolation and Repair Procedure:

Configured in a boxed flow chart format, it represents repetitive faults encountered by our own Service Technicians. Each fault is accompanied by a series of boxes which will ask you to check a particular part of the equipment. After you have carried out the check a question is asked (refer to Fig. 1).

INTERMITTENT -LINE SYNC

Fig. 1

From the result of the question you can then answer YES or NO.

If the answer is NO (as in this case) you must move onto the next box and carry out the next check.

If the answer is YES. the next box will be (refer to Fig. 2).

Fig. 2

Further into the Fault Isolation Procedure you may be asked to check a component: for example (refer to Fig. 3).

Fig. 3

In this case the best method of checking the Tripler Module would be substitution for a serviceable Tripler Module. This would establish very quickly the condition of the Tripler Module, in relation to the fault.

When all the known fault causes have been effected, the chart wil be terminated by a box reading: (refer to Fig. 4).

Fig. 4

If you have reached this stage you should use the circuit diagram and items of test equipment and adopt a logical method of fault isolation and repair.

Illustrated Parts Listing (I.P.L.)

This section contains ail current cabinet designs in the Microvitec 'CUB' monitor range. It is intended to show the service technician how to gain access to the equipment for repair purposes. In addition component part numbers accompany each illustration; this will assist you when ordering a component part.

#### TECHNICAL SPECIFICATIONS

| 1. | ### MODELS - 1431/1432 |  |
| --- | --- | --- |
|  | SYSTEM | 1 : 625 lines, 50 fields interlaced or 31 2/313 lines, 50 fields non interlaced |
|  |  | 2 ; 525 lines, 60 fields interlaced or 262, 263 lines. 60 fields non-interlaced other non-standard systems may be suitable consult MICROVITEC PLC. |
|  | SUPPLY | Nominal 180 - 265V, 40 to 64Hz |
|  | TIMEBASE (LINE) | Pull-in range 15 to 16KHz |
|  | FRAME FREQUENCY | 50Hz - Pull-in range 45 to 65Hz |
|  | POSITIONAL ERROR | + or -3% |
|  | CONVERGENCE ERROR | 0.6mm screen center 1.6mm screen edge |
|  | EHT | 24KV approximately |
|  | EHT REGULATION | + or -1KV |
|  | LINE FREQUENCY | 15.625KHz |
|  | DEGAUSING | Automatic at switch ON |
|  | BANDWIDTH | 18MHz |
|  | RESOLUTION | 452(H)x585(V) elements |
|  | DOT PITCH | 0.64mm |
|  | CRT | Rectangular 335.4mm (screen diagonal) 90 deg deflection. Precision in-line gun vertical stripe screen, high voltage focus |
|  | INPUTS MODEL 1431 | TTL compatible; 1500 ohm R, G, B inputs link selectable, positive or negative video |
|  | MODEL 1432 | Linear; 0 to 4V 1500 ohm R, G, B inputs. Positive video, composite or separate line and field synchronisation - each link selectable positive or negative |
|  | INPUT CONNECTOR | 10 pin in-line 'SYSTEM 25' Pressac |
|  | OPERATING TEMP | 0 to 55 deg C (max) open chassis -10 deg C to +40 deg C (max) |
|  | POWER CONSUMPTION | 65 Watts approximately |
| 2. | ### MODELS 1441 AND 1442 14" HIGH RESOLUTION |  |
|  | NOTE: As models 1431 and 1432, with the following exceptions: |  |
|  | CONVERGENCE ERROR (MAX) | 0.3mm screen center 0.8mm screen edge |
|  | EHT | Approximately 24KV |
|  | BANDWIDTH | 18MHz |
|  | RESOLUTION | 895(H)x585(V) elements, rectangular 333mm (screen diagonal) Automatic degausing at switch on. High focus voltage in-line gun. 90 deg. diagonal deflection, 0.31mm dot pitch black matrix screen with pigmented phosphors. |
|  | DOT PITCH | 0.31mm |
|  | POSITIONAL ERROR | + or -2% |
|  | CONVERGENCE ERROR | 0.8mm screen edge |
| 3. | ### MODEL 1451 |  |
|  | NOTE: As models 1431 and 1432, with the following exceptions: |  |
|  | RESOLUTION | 652(H)x585(V) |
|  | DOT PITCH | 0.43mm |
|  | CONVERGENCE ERROR | 1.2mm screen edge |

| 4. | ### MODEL 2030 |  |
| --- | --- | --- |
|  | NOTE: As models 1431 and 1432, with the following exceptions: |  |
|  | CONVERGENCE ERROR (MAX) | 0.4mm screen center 1.8mm screen edge |
|  | RESOLUTION | 505(H)x585(V) elements, 80 characters on 6 dot, wide matrix |
|  | DOT PITCH | 0.8mm |
|  | CRT | Rectangular 480mm (screen diagonal) 90 deg deflection precision in-line gun vertical stripe screen high voltage focus |
|  | POWER CONSUMPTION | 80 Watts approximately |
| 5. | ### MODEL 2040 |  |
|  | NOTE: As models 1431 and 1432. with the following exceptions: |  |
|  | RESOLUTION | 940(H)x705(V) |
|  | DOT PITCH | 0.47mm |
|  | POSITIONAL ERROR | + or -4% |
|  | POWER CONSUMPTION | 80 Watts approximately |
|  | CONVERGENCE ERROR | 1.0mm screen edge |

##### ELECTRICAL SUPPLY

Warning

THIS APPARATUS MUST BE EARTHED.

Important

The wires in the mains lead are coloured in accordance with the following code:

GREEN AND YELLOW	EARTH

BLUE	NEUTRAL

BROWN	LIVE

As the colours of the wires in the mains lead may not correspond with the coloured markings identifying the terminals in your plug, proceed as follows:

1.      Earth

The wire coloured GREEN and YELLOW must be connected to the terminal marked 'E' or by the safety earth
symbol   	or coloured GREEN, or GREEN and YELLOW.

2.      Neutral

The wire coloured BLUE must be connected to the terminal marked 'N', or coloured BLACK.

3.      Live

The wire coloured BROWN must be connected to the terminal marked ‘L’, or coloured RED.

X-Ray Radiation

X-rays constitute a health hazard on prolonged exposure at dose range unless adequate shielding is provided.

Precautions must be exercised during servicing of display equipment to ensure that the anode voltage and other tube voltages are adjusted to the recommended values.

Replace with a tube of the same type number or recommended replacement to assure continued safety. Implosion Protection

This tube employs integral implosion protection and must be replaced with a tube of the same type or a recommended replacement to assure continued safety.

Shock Hazard

The high voltage at which the tube is operated may be very dangerous. Extreme care should be taken in the servicing or adjustment of any high voltage circuit Caution must be exercised during the replacement or servicing of the tube since a residual electrical charge may be contained on the high voltage capacitor formed by the external and internal conductive coating of the tube funnel. To remove any undesirable residual high voltage charges from the tube, "bleed off" the charge by shorting the anode contact button, located in the funnel of the tube, to the external conductive coating before handling the tube.

Tube Handling

Wear heavy protective clothing, including gloves and safety goggles with side shields. Handle the tube with extreme care. Do not strike, scratch or subject the tube to more than moderate pressure. Particular care should be taken to prevent damage to the seal area.

### OPERATING SAFETY

We recommend — after effecting any Service and/or replacement of any part of the monitor, or after any repair work — that the SAFETY CHECKS listed following are carried out.

###### Note

Ensure on re-assembly that ALL earth connections are replaced SECURELY; in particular safety earths and the 'P' band and dag earth 'CRT' earth connections.

Important!

Failure to observe the points noted could affect your own safety, the product's safety and ultimately, that the user.

### SAFETY CHECKS

Earth Continuity Check

a)      Using a suitable multimeter:

Check between 'Dag' earth on the CRT and mains plug earth pin, (safety earth connection). The reading
should be less than 470Kohm (with tube discharged).

b)      Check mains earth continuity between mains plug earth pin and:

1) CRT 'P' Band

2) PCB 'Earth' pin on main chassis

c)     The final assembly/housing should be checked for continuity between the mains plug earth pin and ALL exposed metalwork.

###### Note

Tests b) and c) should have a resistance of less than 0.5 ohm.

### High Voltage Isolation/Insulation Checks

a)      On the mains supply 3 pin plug, — check between 'live' and 'neutral' to 'earth'  for leakage
breakdown.                                                                                                                              
This test should be made with 1.5KV to 1.6KV AC or DC equivalent.

###### On Test

1) No breakdown should occur.

2) Earth leakage should be less than 6mA at 1.5KV.

b)      Insulation resistance should be greater than 2Megohms at 500V DC.

CAUTION

A MONITOR FAILING ANY OF THE ABOVE CHECKS MUST BE RESTORED TO SAFE WORKING CONDITION BEFORE BEING RETURNED TO THE USER.

SAFETY STANDARDS

SAFETY AND ISOLATION

UNDER NO CIRCUMSTANCES SHOULD ANY FORM OF REPAIR OR MAINTENANCE BE ATTEMPTED BY ANY PERSON OTHER THAN A QUALIFIED ENGINEER.

Switched Mode Power Supply

Although the outputs from the power supply are isolated from the incoming mains supply, the bridge rectifier and the ontrol and regulation circuit ARE NOT isolated. Therefore, when servicing the power supply section of the chassis assembly, the SMPSU should be supplied by a MAINS ISOLATION TRANSFORMER OF AT LEAST 300VA RATING.

HANDLING PRECAUTIONS HT Electrical Charges

The power supply section remains charged with respect to chassis for 30-60 seconds after switching off. Care should be taken when handling the chassis to avoid touching this area during this time.

EHT Charges — Horizontal (Line) Output Stage/CRT

Before handling or attempting adjustment or replacement on the horizontal (line) output stage or on CRT areas — always discharge the final anode of the CRT — by using a suitable EHT probe only. Do this by connecting first to CRT dag earth coating/earth braid assembly, and then to CRT final anode/anode button 'poppy' connector.

Please Note

Very considerable electrical charges can be stored in these areas — even for lengthy periods of time after 'switch off'.

In the interests of your personal safety observe the precautions listed. SAFETY CRITICAL COMPONENTS

Components marked 	on the parts list and circuit diagram are safety approved types and they should be
replaced only with components supplied or approved by our Service Department It is also recommended that the
components not marked with the safety symbol should be replaced by parts of the type originally fitted, and this
applies particularly to those resistors which are stood off the printed circuit boards.

HANDLING PRECAUTIONS — STATIC ELECTRICAL CHARGES

Depending on equipment type and model, the equipment may contain devices which may be damaged by static
electrical charges during handling. Generally, these devices are indicated by a 	symbol.

When replacing or handling these devices or PCBs containing such devices, care should be taken. Soldering irons should be earthed and personnel should use wrist straps earthed via a 1Mohm resistor. If the latter is not practicable they should discharge themselves of any static electricity by touching an earthed point

Static sensitive devices should be packed in suitable conductive containers. Note

Electrostatic discharge does not necessarily 'kill' a component completely — more likely it will 'wound' it

Many static wounded devices will pass normal test both on site and in Service Workshops. In this respect PCBs containing such 'wounded' devices are a major cause of 'no fault found' problems.

It pays, therefore, to cultivate static safe ways when dealing with such static sensitive equipment

______________PRESET CONTROL ADJUSTMENT______________

GENERAL

Preset controls are initially set up at the factory and normally do not require adjustment unless a change is required in the input configuration — for example, typically to install a different graphics adaptor card in the associated host system. Details of the preset controls with their use and adjustment is described following:

PRESET ADJUSTMENTS

TO PROTECT AGAINST ELECTRICAL SHOCK HAZARD AND TO PROTECT THE MONITOR AGAINST SHORT CIRCUIT AND DAMAGE — USE ONLY AN INSULATED NON-METALLIC TRIMMING TOOL TO MAKE ADJUSTMENTS TO THE PRESET CONTROLS.

Care should be taken when adjusting presets. Adjust only one at a time and note carefully the effects of the adjustment before proceeding on to other adjustments. In some cases, it may be advisable to take note of the original setting position of the preset BEFORE adjustment in case the need arises to return to the original setting.

INTERCONNECTION COMPATIBILITY

On installation and prior to preset adjustments, ensure that video and sync connections from the host system are compatible with:

a)      The monitor.

b)      The interconnecting lead assembly in use.

Having determined these points are correct, proceed with the adjustments required according to the details given in the accompanying table and descriptions following.

PRESET CONTROL SETTINGS

1. To set the preset controls, use a signal generating a display occupying as large a screen area as possible. For example a full page of upper case letter 'H' would be suitable, or alternatively a suitable test card as appropriate.

2.      Preset controls in the table following marked with an asterisk * may be adjusted if required.

However, normally this should not be necessary, as these presets are set accurately at the factory during manufacture.

NOTE:   A circle is employed in the screen displays illustrated following, only to demonstrate more clearly the geometric effects of wrong settings.

PRESET LOCATIONS

The physical locations of most of the preset controls referred to in the descriptions following are shown in the illustration contained in the Section preceding.

Notable exceptions are preset controls contained on the TUBE BASE PCB assembly.

The positions of these presets are indicated on the individual PCB's by appropriate ident markings.

| PRESET | WRONG X | RIGHT / |
| --- | --- | --- |
| LINE FREQUENCY |  |  |
| FIELD FREQUENCY |  |  |
| HEIGHT |  |  |
| WIDTH |  |  |
| LINE PHASE* |  |  |
| FIELD LINEARITY* (VERTICAL LINEARITY) |  |  |
| EAST/WEST* CORRECTION |  |  |
| FIELD SHIFT |  |  |

TABLE OF PRESET ADJUSTMENTS

INPUT CONNECTIONS AND CUSTOMER CONTROLS 1.     T.T.L. Compatible/Linear input Selection

A.      T.T.L. compatible or linear (0 to 4V, 1500 ohm) input level options can be selected by moving 3 links: (1)     TL103 R,G,B located on main PCB:

position 1 corresponds with linear levels

position 2 with T.T.L. levels

NOTE: Contrast control VR111, inoperative when position 1 is selected.

2.      Synchronisation - Input Options

A.      Sync inputs must be T.T.L. compatible, the timing of sync pulses should approximately correspond with those used for broadcast purposes (refer to fig. 1 for available options.)

3.     Inverse Video

A.     To facilitate inversion of T.T.L. compatible video input signals by either moving link TL101 on main PCB to position 1, or removing TL101 and feeding a positive T.T.L. level in to pin 9 of PL101.

NOTE: Monitors are normally despatched wired in composite negative (going) syncs mode, with TL102 not fitted, and TL106 selected to position 1.

KEY

1   = +12V

2 = No connection

3 = Sync 3; -field sync

4 = Red video

5 = Sync 2; -field sync

6 = Green video

7 = Sync 1; composite sync or line sync

8 = Blue video

9 = Normal/inverse

T.T.L. video 10 = Ground

Signal input pin connections Fig. 1

4.      Synchronisation Options

| SYNC OPTIONS | INPUTS (PL101) | LINK POSITION |
| --- | --- | --- |
| SYNC OPTIONS | INPUTS (PL101) | LINK POSITION |
| MIXED |  |  |
| -ve going | pin 7 | TL102 not fined |
|  |  | TL106 in position 1 |
|  | pin 7 | TL102 fitted |
| +ve going |  | TL106 in position 1 |
| SEPARATE |  |  |
| -ve line | pin 7 | TL102 not fitted |
| -ve field | pin 5 | TL106 in position 1 |
| +ve line | pin 7 | TL102 fitted |
| +ve field | pin 3 | TL106 in position 2 |
| -ve line | pin 7 | TL102 not fitted |
| +ve field | pin 3 | TL106 in position 1 |

5.      Customer Controls

NOTE: On 'D' series monitors the controls listed below are located on the front of the monitor, concealed behind a downward hinging door.

A.      ON/OFF Switch

(1)     Mounted rear of the monitor, allows mains to be switched on and off without switching the mains supply off.

B.      Contrast/Brilliance Adjustment (VR111)

(1)     Mounted rear of main PCB, only active in T.T.L. mode, and allows video gain to be varied from maximum to minimum, at black level.

C.      Brightness Adjustment (VR134 when fitted)

(1)     Mounted rear of main PCB, next to VR111. Active in all modes of operation allowing brightness of display to be varied above or below cut off.

D.     Volume/Audio Adjustment (when fitted)

(1)    Mounted rear of the monitor, allows user to adjust volume/audio level of the monitor. Clockwise to increase, anti-clockwise to decrease.

E.      Colour Saturation Control (when fitted)

(1)     Mounted rear of the monitor, allows user to adjust the colour level of the monitor. Clockwise to increase, anti-clockwise to decrease.

FACTORY PRESET ADJUSTMENTS

NOTE:   Certain preset adjustments can be from above or below main panel. Adjustments are best made on a static display, preferably a MICROVITEC test card, details of test equipment are available on

application.

1.      Set HT VR4

A.      Adjusted accurately at the factory to give 124V with a dark picture on, R138/137 (right hand side) 5, and should not be re-adjusted.

WARNING:    THIS IS A CRITICAL SAFETY ADJUSTMENT, FAILURE TO COMPLY WITH THE ABOVE WILL INVALIDATE THE WARRANTY.

2.      Line Frequency VR218

A.      Set free running oscillator frequency to almost frequency of incoming line syncs.

B.      Adjust VR218, feed monitor with R,G,B video and interrupt the mixed sync information to line oscillator by removing sync information on PL101, (sync 1, 2 or 3 etc).

C.      Adjust VR218, until picture almost stabilizes then re-connect via PL101 as required. Resulting in a stable picture lock.

3.      Field Frequency VR307

A.     Control of free running field oscillator frequency is achieved by VR307 being adjusted to give a stable picture lock. For effective lock VR307 should be set to centre of locked picture range.

4.     Line Phase VR220

A.     VR220, controls positioning of video information relative to raster in line scan direction. B.      Ensure the following operations have been effected:

(1)	The line frequency has been set (VR218)

(2)	The picture width has been set (L202)

(3)	The monitor is positioned in its place of use.

NOTE: VR220 when adjusted will shift the picture, right or left.

5.     Width L202

CAUTION:      CARE SHOULD BE TAKEN WHEN ADJUSTING THIS COMPONENT DUE TO ITS PROXIMITY TO EHT SECTION. IN PARTICULAR, TRIPLER AND LINE OUTPUT TRANSFORMER.

A.      Using a non-metalic trimming tool adjust L202, to effect picture width adjustment.

6.      Height VR306

A.     VR306. when adjusted will provide for raster under scan and over scan.

7.     Field Linearity VR312

A.      Adjust VR312, to give a linear picture in vertical direction.

NOTE: Best results are obtained by using a cross hatch type grid or MICROVITEC test generator.

8.     Field Shift VR321

A.      VR321, controls positioning of raster in field scan direction.

9.      East-West Correction VR328

NOTE: Some models will not require this adjustment, because east-west correction is integral on certain types of CRT.

A.      Adjustment of VR328 will achieve straight verticals on left and right hand sides of pictures.

10.      Focus

A.     Located on end of tripler module, set brightness control to normal viewing level, then make focus adjustment.

11.      Adjust Colour Background Controls (black level)

NOTE: These controls are factory preset. If adjustment is necessary, an AVO 8 multimeter and/or oscilloscope will be required. However, best results are obtained by using an oscilloscope.

A.     Prepare to adjust colour background controls

(1)     Set customer contrast (VR111), brightness (VR134) and A1 (VR932) fully anti-clockwise

(2)     Disconnect R,G,B, sync inputs

B.     Adjust red, green and blue

(1)    Adjust VR9O6 for red cathode (black level) volts

(2)    Adjust VR914 for green cathode (black level) volts

(3)    Adjust VR921 for blue cathode (black level) volts

(4)    The above voltages are:

(a) 150V-14" monitor (TTL or Linear)

(b) 155V-20" monitor (TTL or Linear)

(c) 140V-12"/14" high res monitor (TTL or Linear)

C.      Adjust A1 voltage

(1)    Adjust VR932 until a raster is just visible.

(2)     Raster colour may be neutral. However, it is very likely shaded towards-red, green, blue or a combination of any two colours.

(3)    Establish raster colour shading as follows:

(a) Red and Green • Yellow

(b) Red and Blue - Magenta

(c) Blue and Green - Cyan

(4)    Reduce black level of remaining one or two guns using VR906, VR914, VR921 or combination until a neutral raster is achieved.

(5)    Re-adjust VR932 to just extinguish raster

(6)    Input - R,G,B and sync signals, then adjust VR111 clockwise

(7)    If correct white balance has not been achieved, repeat operations C. (1) thru (6).

12.    Adjust Colour Gain Controls

CAUTION: MAKE THE FOLLOWING ADJUSTMENTS USING A DC COUPLED OSCILLOSCOPE ONLY. A.      Prepare to adjust colour gain controls

(1)     Disable beam current limit circuit, by removing TL901 in series with CRT heaters on tube base panel.

(2)     Provide a test pattern with peak white and black level information on red, green and blue.

(3)     Ensure VR111 is fully clockwise to provide maximum drive voltages to video output stages.

B.      Adjust red, green and blue gain controls

(3)    Adjust VR903, for red peak to peak drive volts at R926.

(2)    Adjust VR910, for green peak to peak volts at R925.

(3)    Adjust VR916, for blue peak to peak volts at R924.

(4)    Above voltages are:

(a) 70V p-p on 14" monitor - TTL mode

(b) 70V p-p on 20" monitor - TTL mode

(c) 60V p-p on 14"/20" monitor - medium and high resolution

REMOVAL/INSTALLATION DETAILS

WARNING:    ENSURE MONITOR IS DISCONNECTED FROM MAINS ELECTRICAL SUPPLY BEFORE EFFECTING THE FOLLOWING OPERATIONS.

1.      Main PCB Removal/Installation (Fig. 1)

A.      Disconnect EHT lead (1) from CRT, to ensure no charge remains on tnpler connect to chassis metalwork.

B.      Discharge CRT, final anode by connecting it to CRT dag.

C.      Disconnect PL201 (2), PL1 (3) and on cabinet versions PL2 (4) (if fitted) and mains plug PL3.

D.     Remove tube base panel and CRT earthing braid tag.

E.      Remove 'P' band earthing tag (5) from main PCB, located next to tripler module.

F.      Release in turn, each of the nylon self locking PCB support clips (6), lifting PCB slightly in each case.

G.	Disconnect PL101 (7), PL102 (8), then disconnect tube base from CRT.

H.	The main PCB may now be removed by lifting upwards, and withdrawing from the rear.

J.	For installation effect the above operations in reverse order.

Removal/Installation Details Fig. 1

CRT Removal/Installation Fig. 2

2.      CRT Removal/Installation (Fig. 2)

A.     Effect operations 1.A. thru G., then position monitor so that the CRT is face down on two padded support blocks.

B.      Remove two earthing screws (1), then refer to the illustrated mechanical parts section and remove the base of the monitor.

C.	Remove the four nuts (2) and large washers (3) securing CRT to cabinet facia (4).

D.	Carefully withdraw CRT vertically.

E.	Transfer degauss coil (5) and earthing braid (6) and scan coil lead assembly (7) to new CRT.

F.	Install main PCB assembly by effecting operations 1.A. thru G., in reverse order.

G.	CRT installation safety checks.

(1) Check for correct fitting of CRT earthing braid (6).

(2) Ensure black lead from CRT earth braid to tube base panel is connected.

(3) Check 'P' band earth pin has been connected to main PCB.

3.      Line Output Transistor (TR202) and SMPSU Output Transistor (TR2) - Removal/Installation (Fig. 3)

A.       Remove Transistor

(1)  Remove two M3 screws (1) securing transistor and heatsink to main PCB.

(2)  Unsolder base and emitter connections on PC3, then withdraw transistor heatsink (4).

(3}  Separate transistor from heatsink, retaining insulating bushes (3) and mica washer (2) for refitment.

B.      Install Transistor

(1) Coat underside of transistor with thermally conductive heatsink compound.

(2) Effect operations 3.A.(1) thru (3), in reverse order.

Line Output Transistor (TR202) and SMPSU Output Transistor - Removal/Installation

Fig. 3

4.      Tripler - Removal/Installation (Fig. 4)

A.      Tripier Removal

(1)   Remove EHT lead from CRT and discharge CRT final anode to the earth braid.

(2)  Unsolder the following: Lead from line output transformer overwind (1) focus lead (2) to

tube base panel at the tripler end.

(3)  Unsolder from main PCB: Earth return (3) from focus control, clamp diode earth return (4)

(4)  Loosen, but do not remove tripler securing screws (5), then withdraw tripler module (6) from main PCB.

B.     Tripler Module - Installation

(1) Position tripler module (6) on main PCB assembly and secure with screws (5).

(2) Solder on main PCB: Earth return (3) from focus control, clamp diode earth return (4).

(3) Solder	: Lead from line output transformer overwind (1) focus lead (2) to tube

base panel at the tripler end replace insulation sleeve on connection (2).

NOTE: Ensure all soldered connections are smooth and connecting wires kept as short as possible, to guarantee adequate voltage clearances.

(4) Connect EHT lead to CRT final anode.

NOTE: Replace the insulation

sleeve on connection (2)

Tripler Removal/Installation Fig. 4

TYRAP POSITIONS ON CABINET BACK COMPONENTS

Note: All tyraps to be fully tight

___________MODIFICATION FOR IBM/APPLE INTERFACES_________

(1)     Remove TR1 (BC337) and replace with TIP 120 as shown.

(2)     Remove R1 (6 K8) and replace with 8K2 1/4 watt 5%.

(3)     Race a 10 microfarad 35V electrolytic as follows: (i) negative side, to top of existing C4 position (ii) positive side, mechanically wrapped around and soldered to left side of R3.

SWITCH MODE POWER SUPPLY - DESCRIPTION AND OPERATION

The power supply is a variable frequency, self oscillating, switching flyback convertor type, providing mains isolation and three stabilised voltages, of 18V, 124V and 200V.

1.      Control Circuit - Description

A.      Operating Normally In A Ready State

(1)     TR2 turns on a step voltage whose amplitude depends on the instantaneous value of rectified mams across T2 primary.

(2)     Current in winding and TR2 collector increases in a linear fashion from zero, in which time energy is stored as flux in the transformer.

(3)    During this time D22, D23 and D24 are reverse biased and any load energy is supplied by C27, C28. C31 and C26, from previous cycle.

(4)    TR1 performs a control function, supplied from a reference winding on transformer (nominal +30V).

(5)    During 'ON' time of TR2. the emitter of TR1 is held at a constant with regard to reference rail from D18.

(6)    The base of TR1 is fed directly from the reference rail, via R3, VR4 and R5, any voltage change on the reference arising from a change of voltage at the main output will vary causing the constant current source, used to charge C16, to vary in sympathy.

(7)    C16 charges on the current available from R10. The voltage across C16 increases until it reaches the gate trigger voltage of TY1. Then TY1 conducts and 'crow bars' base drive to TR2.

(8)    TR2 ceases conduction and its collector voltage becomes positive. The dV/dT at TR2 collector, is limited by C17, R12 and D17. As this occurs D22. D23, D24 and D21 become forward biased and stored energy within the transformer is transferred into output capacitors, and their respective loads.

(9)    Eventually voltages on D6, D21. D22. D23 and D24 anodes, collapses. TY1 is forced off prior to this stage by negative anode voltage, allowing TR2 to turn on. Full base drive is then sustained by R16.

TR2 BASE Fuse in

TR2 BASE START-UP Fuse out

(10)  HT stabilisation

(a) HT stabilisation is achieved by controlling the duty cycle of the switching transistor.

(b) Increasing load = increased duty cycle and peak collector current.

(c) Increasing mains supply * increased operating frequency.

(d) HT adjustment is made by VR4.

(e) Extra damping for C14. R6 and D7. are required to limit Vce of TR2. 015 and D16 provide negative off drive and base current tracking, L2 optimises storage time of TR2 for minimum switching losses.

(11) Max available power is determined by measuring, peak collector current of TR2, and is sensed by R15. If voltage across R15 exceeds voltage across TY1 gate cathode, then TY1 conducts turning TR2 off.

NOTE: This sequence occurs during start-up, at low mains and under fault conditions. 2.     Start-Up Procedure

A.      Current required at start-up is small compared with base drive current under normal operating conditions.

B.     When turn on of TR2 occurs  in this manner (once every 20ms) the oscillation becomes self sustaining. R8 continues to supply current, but is swamped by forward base drive from R16.

TR2 COLLECTOR Fuse in

3.       Over Voltage Protection

A.     Controlled by a second feedback loop attached to TY1, consisting of:

(1)    A zener diode which senses the reference rail voltage and HT voltage proportionally.

(2)    If this reference exceeds zener voltage. 020 conducts, fires TY1 and terminates drive to TR2. During which time enough volts are developed across TY2 gate-cathode to cause conduction, and latch on.

(3)    Drive to TR2 is stopped until C23 is fully discharged (10ms).

(4)    Power supply is off until the next mains start-up pulse.

4.     Short Circuit Protection

A.     Short circuit or over current on any output rail represents an increase in stored energy

required from T2. therefore an increase in collector current through TR2. detected by R15. Thus TY1 is fired and TR2 turned off.

B.      Now operating in 'Burst Mode', that is, the power supply is initiated under normal start-up conditions, but only operates for a few switching cycles when the over current protection comes into operation terminating drive to TR2.

VIDEO INPUT INTERFACE CIRCUIT - DESCRIPTION AND OPERATION

INPUT: Connections are made at PL101-red, green and blue video, sync options 1, 2, 3 and TTL video

normal/invert. All inputs are flashover protected by, resistors and diodes. R,G,B drives are split in two ways:

1.      Test selectable links TL103, R,G,B.

2.      To IC101

Select Position  1: The input stage is in the linear mode, the video is buffered and level shifted by emitter output stages. These provide temperature tracking with TR103, 104 and 105 resulting in a stable black level.

NOTE: In the linear mode only brightness variations of the video information are possible using VR314.

Select Position 2: TR103, 104 and 105 bases are driven by IC101. This option is used when driving from TTL video sources offering primary/secondary colour and black and white drives.

NOTE: In the TTL mode, signal to noise immunity of system is very good. IC101 can be used with negative TTL level video drives (R,G,B), in order to invert video information.

Video inversion is achieved by toggling video polarity select line at. PL101 pin 2 or TL101:

Normal -   0V Invert   -  + 5V

TTL Mode: Contrast of video information is tracked by varying available potential across open collector load resistors, R114, 115 and 116 which are supplied by TR101 from +12V, the base being driven from the contrast control slider VR111, R112. C101 form a low pass fitter and ensure smooth operation of contrast control.

CRT Beam Current: Information is fed to D117, 118, from a constant current source derived from 124V main HT rail. As CRT beam current increases D117, 118 junctions become more negative thus; D117 conducts more heavily causing voltage to TR106 base to decrease. R136, C105 filter the signal, the derived voltage is emitter followed and supplies TR103, 104 and 105 emitters directly. Hence increases in CRT beam current, above a preset limit achieving an automatic reduction in picture brightness.

NOTE: CRT beam limiting is preset depending on monitor model.

Brightness of display in all modes is adjusted by. VR134 enabling parallel adjustment of R,G,B and black levels within a + or -20V range from nominal:

400uA - High and medium resolution 700uA - 14" standard resolution TTL/Linear 900uA - 20" standard resolution TTL/Linear

TR102: A fast switching transistor used to derive mixed blanking pulses for flyback blanking of video information. The base is driven from a potential divider/mixer network, from a line flyback pulse and a frame flyback pulse. Line flyback is advanced in phase with C225, to allow for transistor switching delays. D107, holds TR102 in a semi-saturation state.

TR102 COLLECTOR with 10 to 1 scope probe

IC101 is also used to provide the following sync options:

Composite negative sync: Fed in on PL101, pin 7, ensure TL102 in non-active position, allowing pin 2 of IC101, to be pulled high where upon IC101 performs a sync inversion and provides an attenuated positive sync waveform for driving sync separators of IC201, via R201 and TL106.

Composite positive sync: Fed in on PL101, pin 7 with TL102 in its grounded position. IC101, now provides an output in phase with input and of suitable amplitude for driving IC201 directly, via R201 and TL106.

Separate negative line and field syncs: Ensure TL102 in non-active position, line syncs are fed in on PL101, pin 5. IC101, performs an exclusive OR function, the output being an inverted composite sync waveform.

Separate positive line and field syncs: Fed in on PL101, pin 7 in its grounded position. TL106, is switched over to inverse field option. IC101, provides an attenuated and buffered line sync feed for IC201, via R201. Positive field sync information is fed directly PL101, pin 3 "sync3" input by, TL106(B) and R202.

LINE TIMEBASE - DESCRIPTION AND OPERATION

1.       Line Timebase

A.      Line oscillator function is based on IC201, providing three outputs:

(1)     Horizontal drive pulses for control of line output stage.

(2)     Vertical sync pulses compatible with synchronisation of IC301 field output IC.

(3)     A sandcastle pulse providing burst gate and clamping information.

NOTE: This facility is only used with IC TDA3301, currently required by '1 volt 75 ohm' linear/PAL input monitors.

2.      Sync Separator

A.      IC (TDA1180P), incorporates separate noise gated sync separators for line/field syncs, which accepts positive going sync pulses (or negative going composite video) on pins 8 and 9.

B.     Output pulses from the line sync separator are used in conjunction with a sync gate to synchronise line oscillator in a phase locked loop circuit.

3.      Line Oscillator - Phase Detectors

A.      The line oscillator is timed by a network of resistors and capacitors on pins 14 and 1 5 of IC201, used to derive a pulse of suitable mark space ratio for driving line output stages.

B.      IC201 contains two basic control loops, each containing a phase detector.

(1) The first phase detector compares output of the line oscillator with the incoming line sync pulse. Phase detector output on pin 13, is filtered and fed to the voltage control input of the oscillator on pin 15.

(2)    The second phase detector, compensates for delays introduced by the line output stage and compares line flyback pulses at pin 6, with oscillator output. Phase detector output consists of a bi-directional current source used to charge/discharge C213 on pin 5. Voltage derived from C213 is used to control a phase shifter, which regulates the phase of the output pulse on pin 3. Pin 5 also provides a 'line shift' function, by offsetting voltage developed across C213. charged from VR220, R221 and R222 allowing phase shift of + or -1uS, between line scan and video information.

(3)    A 7uS gate pulse from the line oscillator, whose phase position is centred around the horizontal sync pulse. The gated pulse is used to control the arrival of sync pulses at the sync phase detector for a duration of 7uS, allowing latching and de-latching of line oscillator. Obtained by a coincidence detector which compares the phase gate pulse with that of incoming syncs.

(4)    When the two signals are not aligned, the coincidence detector is used to switch p.l.l. filter into a short time constant mode, giving a high input impedance at pin 12, thus increasing sensitivity and loop gain of oscillator. The phase locked loop now has a low noise immunity but has a very wide capture range. When aligned coincidence detector activates the time constant switch, causing low impedance on pin 12, achieving a lower sensitivity and loop gain, but providing a high degree of noise immunity. During the 'locked' condition the p.l.l. operates with a long time constant.

NOTE: A short time constant mode, can be achieved manually by connecting the output of the coincidence detector on pin 11, to ground. Allowing the oscillator to follow rapid fluctuations in line period, which may occur on some non-standard signals.

4,      Line Driver Stage

A.      Horizontal drive pulses from pin 3 (IC201), are D.C. coupled to TR2Q1 and used to control driver transformer (T201), providing the impedance conversion necessary to provide 600mA forward base current, for saturation of line output transistor (TR202). Ringing is damped by R225 and C214 at TR201 turn off, thus limiting its Vce to a safe value. HT supply to the line

driver comes from main HT supply rail, prior to R231 and HT scan interlock (PL201 pins 5/6; allowing its operation to be checked independently of the line output stage.

IC201 PIN 3 with 10 to 1 scope probe

TR202 COLLECTOR with 100 to 1 scope probe

5.      Vertical Sync Output

A.      Output of field sync separator is used to drive vertical sync output stage on pin 10 (IC201).

B.      In addition, this pulse is used internally to inhibit the first phase detector during the field sync period, thus preventing 'top flutter' as a result of equalising pulses.

6.      Sandcastle Pulse

A.      Sandcastle pulse is on pin 7 (IC201), used on models with linear interface PCB assembly consisting of two sections.

(1)     Upper portion, suitable for burst gate and clamping operations from the horizontal oscillator, thus ensuring an accurate phase relationship with the video information.

(2)    Lower portion, derived from a line flyback 'slice' for line blanking

LINE OUTPUT - DESCRIPTION AND OPERATION A.      Line Output

1.      L202. L203 and T202 primary, are tuned during the flyback period by C222. This lasts for 11.8/11. 1uS, on 14"/20" monitors.

2.       Line output transistor TR2C2, is driven directly from the secondary winding of T201. 'ON' current is controlled by R227, turn off dissipation is minimised by L204.

3.       Line linearity correction is provided by L203, which is damped by C217, R230 'S' correction is provided by C218.

4.      Field timebase +25V (IC301), is achieved by rectifying a negative going flyback voltage from a secondary winding on line output transformer. A fusable resistor provides CRT protection under possible fault conditions.

LINE SYNC with 10 to 1 scope probe

TR202 COLLECTOR with 100 to 1 scope probe

B.      EHT Supply

1.      23.5kV required for CRT is generated by a tripler module driven from a 7.5kV, overwind on T203. Inductance of the transformer (between primary and overwind), is tuned to the 7th

harmonic of the flyback frequency by tripler input capacitance and self capacitance of the overwind.

2.      The 'breathing' performance of the display is further improved by deriving a high focus

potential from a resistive thick film/substrate potential divider from the EHT. giving rise to a constant bleed current from EHT, thus lowering output impedance of the EHT circuit.

3.      An extra input diode within the tripler has its anode connected to the tube base ground and via a beam current sensing circuit to 0V. C223 and a network of resistors provide a load for the diode and effectively damp out ringing which may occur during scan. The resulting 1000V which occurs across C223 is used to generate A1 potential across CRT.

C.       HT Supply

1.      Derived from the main secondary winding of the switch mode power supply via R231. R231 is chosen to optimise picture breathing performance and offer protection to TR202, during CRT flashover.

FIELD TIMEBASE* - DESCRIPTION AND OPERATION

The field timebase -..":• ;^   s carried out by  ;-.:2C*    2>:     -.-: - --_ -. ^~~ :.   s determined by VR307, R308 and C303. produ; ~: j  '2C.S   : .:c .: -'- 3'':  at pin 12   - •? -  ; • .-.-;;:-   s  :?rved from potential across C305 and C306 as •--••.   :-ange towards -25Vsupp .   . -j - 11-   R305 R306 and R301. When field scan is completed the  ' COuS pulse discharges C305 and C306  ready for charging cycle to be restarted.

1.       Field Linearity

A.       Sawtooth output on pin ? of IC301 via R?13 and ???? field linearity control.

IC301 PIN  1     with 10 to 1 scope probe

2.      Field Flyback

A.       In order to achieve a short fieid flyback time, a supply voltage larger than required during

scan, must be applied to field deflection coils during flyback period. Made possible by using a separate field flyback generator, within IC301.

B.      Main HT supply for IC301 is supplied to pin 5 via D302. During flyback the generator doubles the supply on pin 5, the potential on pin 3 is switched from 0V during scan to +25V during flyback The change in voltage occurs on pin 5 via C304 causing potential to double during flyback.

C.      D302 isolates pin 5 from +25V supply. When deflection coil field has collapsed and potential across field scan coils has fallen below  +25V pin 3 is switched back to 0V and scan cycle resumed.

D.      Synchronisation of IC301 is achieved by feeding a positive going) field sync pulse on pin 8 of IC301

!C301 PIN 8 with 10 to 1 scope probe

3.      HT Supply for IC301

A.      Derived from a scan rectified rail from the line output stage. C305, C306 are fed via VR306 (height control) from scan rectified supply and 12V rail. Proportions of current and associated time constants R303, C301 and D301 are used to minimise "picture bounce" thus maintaining accurate tracking of the field scan with line scan amplitude during CRT beam current variations, therefore reducing picture "breathing" effects.

4.      Field Output

A.      Sawtooth output on pin 1 is applied to output stage (within IC301) and scan output is

available from pin 4 to field deflection coils. Current within coils is sampled by R323, then fed back via R31 7 to the virtual earth input pin 10 of IC301.

B.      Gain of output amplifier is set by the ratio of R314 and R31 7. DC operating point by R318 and R316.

IC301 PIN 4 with 10 to 1 scope probe

5.      Picture Geometry

A.     CRT E-W pincushion distortion is corrected by modulation of line deflection current in

transductor (T202) actively driven by TR301. which is then fed from the parabolic waveform at the top of the S correction capacitor C311. AC gam from the amplifying driver is used to control amount of correction applied to CRT.

TR301 COLLECTOR with 10 to 1 scope probe

CRT TUBE BASE PANEL - DESCRIPTION AND OPERATION

1.       CRT Tube Base Panel (Refer to fig. 1)

2.       Circuit Description

A.      All CRT electrodes are protected by a resistor, capacitor and spark gap.

(1)     Scark gaps on all electrodes (except focus) are formed by a 1 -2kV ring trap gap,

positioned within CRT base socket assembly. High focus voltage has a separate 10kV spark gap contained within tube base socket.

(2)     CRT cathodes are stood off from video outputs by 220 ohm resistors, the grid 100k and A1's 820k.

(3)     Decoupling of grid and A1's is achieved by C910 and C911.

(4)     Focus voltage is provided by a potential divider located within tripler module, providing an adjustable voltage of 5-8KV.

(5)    A1 voltage is adjusted by VR932, offering a range of 350-820 volts.

(6)     CRT heaters may be disconnected by removing TL901, in order to make video adjustments.

(7)     CRT cathodes are directly driven from video output stages mounted on CRT panel.

NOTE: The component values given in this section refer to a standard 14 inch model. For equivalent values, refer to the parts listing.

3.      Video Output Stages - Circuit Description and Operation

NOTE: Red, Green and Blue video outputs are identical, the following text refers to the red output stage.

A.     TR902 forms a class 'A' amplifier, AC gain is derived from the ratio of R935 to R902. VR903 and DC gain by a DC offset current from R905 and VR906.

B.      R904 forms video output load and TR902 represents a low impedance drive source to CRT input capacitance during its conduction.

C.      During turn off of TR902. the source impedance of the load R904 is considerably reduced by TR901, ensuring a good 'pull-up' performance.

D.      Video compensation is achieved by split capacitances, C902, C903 to help maintain a constant amplifier response curve over the full range of VR903.

E.      The emitters of TR902. TR904 and TR906 are connected together with a DC reference of approximately 7.5V, used to set video black level voltage.

F.      TR907 performs line and field blanking of video information.

(1)    TR907 is driven by negative (going) mixed blanking pulses from TR102. TR907 conducts providing a 7.5V black level reference.

(2)     During line and field flyback TR907 is turned off, forcing video outputs off.

(3)     Beam current information is sensed on tube base panel resistively, across line output ground to 0V line by R937, D117, D118 on main PCB.

PARTS LISTING SERIES 3 SERVICE MANUAL

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| R1 |  | RK105GKO | 100K CARBON COMP. RESISTOR 10% 0.5W AXIAL |
| R2 |  | RF185JJO | 180K CARBON FILM RESISTOR 5% 1W AXIAL |
| R3 |  | RF393DJO | 3K9 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R3 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1449/AS) | RF273DJO | 2K7 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R3 | (1496/LI2U) |  |  |
|  | (1486/L12U) | RF363DJO | 3K5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R3 | (1241 /AS) |  |  |
|  | (1446/L12) |  |  |
|  | (1451/MQ3) | RF473DJO | 4K7 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R3 | (1446/LF) | RF333DJO | 3K3 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR4 |  | RQ103AL2 | 1K PRESET POT CARB. MIN. H MTG 20% 0.1W AXIAL |
| R5 |  | RF104DJO | 10K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R5 | (1441/AL) |  |  |
|  | (1441 /AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1449/AS) | RF683DJO | 6K3 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R5 | (1486/LI2U) | RF323DJO | 8K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R6 |  | RW154XJ 6 | 15K WW RESISTOR 5% 7W RADIAL |
| R7 |  | RF471DJO | 47R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R8 |  | RF333JJO | 3K3 CARBON FILM RESISTOR 5% 1W AXIAL |
| R8 | (1496/L12U) |  |  |
|  | (1486/LI2U) | RF224GJO | 22K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R8 | (1451/AS) | RF564GJO | 56K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R10 |  | RF392DJO | 390R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R11 |  | RF221DJO | 22R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R12 |  | RW103XJ5 | 1K WW RESISTOR 5% 9W RADIAL |
| R12 | (1451AP/DS) |  |  |
|  | (1451/DS) | RW103VJ5 | 1K WW RESISTOR 5% 7W RADIAL |
| R13 |  | RF274DJO | 27K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R14 |  | RF102DJO | 100R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R15 |  | R0150LJO | 1R5 METAL OXIDE RESISTOR 5% 2W AXIAL |
| R16 |  | RW472ZJ5 | 470R RESISTOR 5% 1 1W RADIAL |
| R16 | (1496/LI2U) |  |  |
|  | (1486/L12U) | RW103XJ5 | 1K WW RESISTOR 5% 9W RADIAL |
| R16 | (1451/AS) | RW333RJO | 3K3 WW RESISTOR 5% 5W RADIAL |
| R16 | (1451AP/DS) |  |  |
|  | (1431/DS) |  |  |
|  | (1451/DS) | RW103VJ5 | 1K WW RESISTOR 5% 7W RADIAL |
| R17 |  | RG336GJO | ! 3M3 METAL GLAZE RESISTOR VDE/BS415 APPROVED |
|  |  |  | 5% 0.5W AXIAL |
| R18/20 |  | RF394GJO | 39K CARBON FILM RESISTOR 5% 0.5 AXIAL |
| R21 |  | RF392DJO | 390R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R22 |  | RF682DJO | 680R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R23 |  | RF104DJO | 10K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R23 | (1441/AL) | RF683DJO | 6K8 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R24 |  | RF224DJO | 22K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R25 |  | R0470UO | 4R7 METAL OXIDE RESISTOR 5% 2W AXIAL |
| R26 |  | RL470GJO | ! 4R7 FUSIBLE METAL RESISTOR 5% 0.5W AXIAL |
| R27 |  | RF125DJO | 120K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R29 |  | RF185DJO | 180K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R29 | (1486/LI2U) | RF684DJO | 68K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R30 |  | WL2214TU1 | LINK 5mm x 14mm x 5mm |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| R32 | (1431AP/MS) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1451AP/DS) |  |  |
|  | (2040/CS5) | RF392DJO | 390R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R101 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R102-107 |  | RF152DJO | 150R CARBON FILM RESISTOR 5% 0.25VV AXIAL |
| R108 |  | RF474DJO | 47K CARSON FILM RESISTOR 5% 0.25W AXIAL |
| R110 |  | RF474DJO | 47K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR1 1 1 |  | KP0025A05 | PLUG 5-WAY 20/3445 |
| VR11 1 | (1451/AS) |  |  |
|  | (1441/AS) |  |  |
|  | (1241/AS) | RQ104CL1 | 10K PRESET POT CARB MIN 20% 0.15W V.MTG |
| VR111 | (1439/AS) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1439/MS) | KP0026A08 | PLUG 8-WAY PRESSAC 2S/20/3448/BDFH |
| R112/113 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R114-116 |  | RF472DJO | 470R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R117 |  | RF632DJO | 53CR CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R118 |  | RF222DJO | 220R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R120-123 |  | RF153DJO | 1K5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R124 |  | RF472DJO | 470R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R125 |  | RF153DJO | 1K5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R126 |  | RF103DJO | IK CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R127/128 |  | RF222DJO | 220R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R130 |  | RF222DJO | 220R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R131-133 |  | RF102DJO | 100R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR134 |  | RQ103AL2 | 1K PRESET POT CARBON MIN 20% 0.1W H.MTG |
| R135/R136 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R137 |  | RF185DJO | 180K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R138 |  | RF105DJO | 100K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R138 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1496/LI2U) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451AP/MS4 |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | RF1850JO | 180K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R140 |  | RF474DJO | 47K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R201 |  | RF273DJO | 2K7 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R202 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R203 |  | RF686DJO | 6M8 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R204 |  | RF156DJO | 1M5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R205 |  | RF226DJO | 2M2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R206 |  | RF394GJO | 39K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R207 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R208 |  | RF101DJO | 10R CARBON FILM RESISTOR 5% 0.25W AXIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| R210 |  | RF825DJO | 820K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R211 |  | RF123DJO | 1K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R212 |  | RF363DJO | 3K5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R213 |  | RF105DJO | 100K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R214 |  | RF225GJO | 220K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R215 |  | RF104DJO | 10K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R215 | (1441/AL) | RF913DJO | 9K1 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R216 |  | RF324DJO | 82K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R217 |  | RF224DJO | 22K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR218 |  | RQ224AL1 | 22K PRESET POT CARBON MIN 20% 0.1W V.MTG ¦ |
| VR220 |  | RQ225AL1 | 220K PRESET POT CARBON MIN 20% 0.1W V.MTG |
| R221 |  | RF225DJO | 220K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R222 |  | RF155DJO | 150K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R223/224 |  | RF222DJO | 220R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R225 |  | RW333LJ5 | 3K3 WW RESISTOR 5% 2W (PLUG) |
| R226 |  | RX223PJO | ! 2K2 FUSIBLE WW RESISTOR 5% 2W RADIAL |
| R227 |  | RF100JJO | 1RO CARBON FILM RESISTOR 5% 1W AXIAL |
| R227 | (1496/L12U) |  |  |
|  | (1431/DS) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (203 VAX) |  |  |
|  | (2032/AM) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/MS) |  |  |
|  | (1451/DS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1241/AT) | R0100JJO | 1R0 METAL OXIDE RESISTOR 5% 1W AXIAL |
| R228/230 |  | RW682PJO | 680R WW RESISTOR 5% 4W AXIAL |
| R231 |  | RX1518J6 | ! 15R FUSIBLE WW RESISTOR 5% 2W8 RADIAL |
| R232 |  | R0101LJO | 10R METAL OXIDE RESISTOR 5% 2W AXIAL |
| R233/234 |  | RF394GJO | 39K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R235 |  | RL101GJO | ! 10R FUSIBLE METAL RESISTOR 5% 0.5W AXIAL |
| R236 |  | RF155JJO | 150K CARBON FILM RESISTOR 5% 1W AXIAL |
| R237 | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) | RW220PJO | 2R2 WW RESISTOR 5% 4W AXIAL |
| R301 |  | RF685DJO | 680K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R302 |  | RF124DJO | 12K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R303/304 |  | RF475DJO | 470K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R305 |  | RF685DJO | 680K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR306 |  | RQ475AL1 | 470K PRESET POT CARBON MIN 20% 0.1W V.MTG |
| VR307 |  | RQ105AL1 | 100K PRESET POT CARBON MIN 20% 0.1W V.MTG |
| R308 |  | RF155DJO | 150K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R310 |  | RF183DJO | 1K8 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R311 |  | RF563DJO | 5K6 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR312 |  | RQ105AL1 | 100K PRESET POT CARBON MIN 20% 0.1 W V.MTG |
| R313/314 |  | RF474DJO | 47K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R314 | (1241/AS) | RF155DJO | 150K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R314 | (1441/AL) | RF3340JO | 33K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R315 |  | RF225DJO | 220K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R316 |  | RF184DJO | 18K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R316 | (1241/AS) | RF394DJO | 39K CARBON FILM RESISTOR 5% 0.25W AXIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| R317 |  | RF563DJO | 5K5 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R317 | (1241/AS) | RF154DJO | 15K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R318 |  | RF184DJO | 18K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R313 | (1241/AS) | RF274DJO | 27K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R320 |  | RF330DJO | 3R3 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR321 |  | RS103PL2 | 1K PRESET POT CARB MIN WW H.VTG 20% 4W |
| R322 |  | RF332JJO | 330R CARBON FILM RESISTOR 5% 1W AXIAL |
| R323 |  | RF100DJO | 1R0 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R323 | (1441/AL) | RF333DJO | 3R3 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R324 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R325 |  | RF182JJO | 180R CARBON FILM RESISTOR 5% 1W AXIAL |
| R326 |  | RF152GJO | 150R CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R327 |  | RF331DJO | 33R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR328 |  | RQ222AL2 | 220R PRESET POT CARB MIN 20% 0.1W H.MTG |
| R801 |  | R0154LJO | 15K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R802 |  | RL102GJO | ! 100R FUSIBLE METAL FILM RESISTOR 5% 0.25W AXIAL |
| R803/804 |  | RF224GJO | 22K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R805 |  | RF183DJO | 1K8 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR806 |  | RQ103AL2 | 1K PRESET POT CARS MIN H MTG 20% 0.1 W |
| R807 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR808 |  | RQ473AL2 | 4K7 CARSON POT CARB MIN MTG 20% 0.1W |
| R309 |  | RF331DJO | 33R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R810 |  | RF103DJO | 10K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R811 |  | RF472DJO | 470R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R811 | (1441) |  |  |
|  | (1442) | RF331DJO | 33R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R812 |  | R0154LJO | 15K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R813 |  | RL102GJO | ! 100R FUSIBLE METAL FILM RESISTOR 5% 0.25W AXIAL |

R

| 814/815 |  | RF224GJO | 22K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| --- | --- | --- | --- |
| R816 |  | RF183DJO | 1K8 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR817 |  | RQ103AL2 | 1K PRESET POT CARB MIN H MTG 20% 0.1W |
| R818 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R819 |  | RF331DJO | 33R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR820 |  | RQ473AL2 | 4K7 PRESET POT CARB MIN H MTG 20% 0.1W |
| R821 |  | R0154LJO | 15K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R822 |  | RL103GJO | ! 1K FUSIBLE METAL FILM RESISTOR 5% 0.25W AXIAL |
| R822 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS |  |  |
|  | (1442 ALL MODELS) |  |  |
|  | (1241/AS) | RL102GJO | ! 100R FUSIBLE METAL FILM RESISTOR 5% 0.25W AXlAL |
| R823/824 |  | RF224GJO | 22K CARBON FILM RESISTOR 5% 0.5W AXIAL |
| R825 |  | RF183DJO | 1K8 CARBON FILM RESISTOR 5% 0.5W AXIAL |
| VR826 |  | RQ103AL2 | 1K PRESET POT CARB MIN H MTG 20% 0.1W |
| R827 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR828 |  | RQ473AL2 | 4K7 PRESET POT CARB MIN H MTG 20% 0.1W |
| R829 |  | RF331DJO | 33R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R830/831/832 |  |  |  |
| 833 |  | RK222GKO | 220R CARBON COMP RESISTOR 10% 0.5W AXIAL |
| R833 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1442 ALL MODELS) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) | RK105GKO | 100K CARBON COMP RESISTOR 10% 0.5W AXIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| R824 |  | RF273DJO | 2K7 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R835 |  | RF475DJO | 470K CAPBON FILM RESISTOR 5% 0.25W AXIAL |
| R836 |  | RK825GKO | 820K CARBON COMP RESISTOR 10% 0.5W AXIAL |
| VR837 |  | RQ2260L2 | 2M2 PRESET POT CARB H MTG 20% 0.15W |
| R838 |  | R??????0 | 180K CARBON FILM RESISTOR 5% 1W AXIAL |
| R840 |  | ??????? | 150K CARBON FILM RESISTOR 5% 1W AXIAL |
| VR901 |  | RQ103AL1 | 1K PRESET POT CARB MIN - MTG 20% 0.1W AXlAL |
| R901 |  | RF103DJO | 1K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R902 |  | RF223DJQO | 2K2 PRESET POT CARB MIN H MTG 20% 0.1W AXIAL |
| VR902 |  | RF222DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR903 |  | RQ102AL2 | 1K PRESET POT CARB MIN H MTG 20% 0.1W |
| R904 |  | R0154LJO | 1 5K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R905 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR906 |  | RQ473AL2 | 4K7 PRESET POT CARB MIN H MTG 20% 0.1W |
| R907 |  | RL102GJO | ! 100R FUSIBLE METAL FILM RESISTOR 5% 0.5W AXIAL |
| R908 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR910 |  | RQ103AL2 | 1K PRESET POT CARB MIN H MTG 20% 0.1W |
| R911 |  | RF474JJO | 47K CARBON FILM RESISTOR 5% 1W AXIAL |
| R912 |  | RG154LJO | 1 5K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R913 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR914 |  | RQ473AL2 | 4K7 PRESET POT CARB MIN H MTG 20% 0.1W |
| R915 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR916 |  | RQ103AL2 | 1K PRESET POT CARB MIN H MTG 20% 0.1W |
| R917 |  | RF474JJO | 47K CARBON FILM RESISTOR 5% 1W AXIAL |
| R918 |  | R0154LJO | 15K METAL OXIDE RESISTOR 5% 2W AXIAL |
| R920 |  | RF223DJO | 2K2 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| VR921 |  | RQ473AL2 | 4K7 PRESET POT CARB MIN H MTG 20% 0.1W |
| R922 |  | RL102GJO | ! 100R FUSIBLE METAL FILM RESISTOR 5% 0.5W AXIAL |
| R923 |  | RF472DJO | 470R CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R924-926 |  | RK222GKO | 220R CARBON COMP RESISTOR 10% 0.5W AXIAL |
| R927 |  | RK105GKO | 100K CARBON COMP RESISTOR 10% 0.5W AXIAL |
| R928 |  | RF475DJO | 470K CARBON FILM RESISTOR 5% 0.25W AXIAL |
| R931 |  | RK825GKO | 820K CARBON COMP RESISTOR 10% 0.5W AXIAL |
| VR932 |  | RQ226CL2 | 2M2 PRESET POT CARBON H MTG 20% 0.15W |
| R933 |  | RF155JJO | 150K CARBON FILM RESISTOR 5% 1W AXIAL |
| R933 |  | RF185JJO | 180K CARBON FILM RESISTOR 5% 1W AXIAL |
| R934 |  | RF185JJO | 180K CARBON FILM RESISTOR 5% 1W AXIAL |
| R934 |  | RF225JJO | 220K CARBON FILM RESISTOR 5% 1W AXIAL |
| R935 |  | RF474JJO | 47K CARBON FILM RESISTOR 5% 1W AXIAL |
| R936 |  | RL102GJO | ! 100R FUSIBLE METAL RESISTOR 5% 0.5W AXIAL |
| R937 |  | RF273DJO | 2K7 CARBON FILM RESISTOR 5% 0.25W AXIAL |
| C1 |  | CX225NL6 | ! 220nF 250V AC MET POLY CLASS X CAP 20% RADIAL |
| C2/3 |  | CY103NL6 | ! 1000pF 250V AC CERAMIC DISC CAPACITOR CLASS Y |
|  |  |  | 20% RADIAL |
| C4 |  |  | AS C1 |
| C5 |  | CD472YL6 | 470pF 2KV CERAMIC CAPACITOR 20% RADIAL |
| C6 |  | CM104TL6 | 10nF 630V MET POLY CAPACITOR 20% RADIAL |
| C7 |  | CD472YL6 | 470pF 2KV CERAMIC CAPACITOR 20% RADIAL |
| C8 |  |  | AS C6 |
| C10 |  | CM554RK6 | 56nF 400V POLY CAPACITOR 20% RADIAL |
| C10 | (1496/LI2U) |  |  |
|  | (1486/LI2U) | CM105NL6 | 100nF POLY CAPACITOR 20% RADIAL |
| C11 |  | CA108RM7 | 100uF 385V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C11 |  | CA288QM7 | 220uF 385V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C12 |  | CA476HM6 | 4.7uF 35V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C13/14 |  | CM105RL6 | 100nF 400V MET POLY CAPACITOR 20% RADIAL |
| C15 |  | CD473KK6 | 4700pF 100V CERAMIC CAPACITOR 20% RADIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| C16 |  | CR104MKO | 10nF 160V POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C17 |  | CL223XJ6 | ! 2200pF 2000V POLYPROPYLENE CAPACITOR 5% RADIAL |
| C13 |  | CA106JL0 | 1uF 50V ALUM ELECT CAPACITOR 20% AXIAL |
| C20 |  | CY103NL6 | ! 1000pF 250V AC CERAMIC DISC CAPACITOR CLASS Y |
|  |  |  | 20% RADIAL |
| C21 |  | CM1C5RL6 | 100nF 400V POLY CAPACITOR 20% RADIAL |
| C22 |  | CD223FL6 | 2200pF 25V CERAMIC CAPACITOR 20% RADIAL |
| C23 |  | CA476HM6 | 4.7uF 35V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C24 |  | CM105NL6 | 100nF 250V MET POLY CAPACITOR 20% RADIAL |
| C25 |  | CD472RL6 | 470pF 400V CERAMIC CAPACITOR 20% RADIAL |
| C26-28 |  | CA477NM6 | 47uF 250V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C30 |  | CD472RL6 | 470pF 400V CERAMIC CAPACITOR 20% RADIAL |
| C31 |  | CA478FM6 | 470uF 25V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C32,33 |  | CM105ML6 | 100nF 160V POLY CAPACITOR 20% RADIAL |
| C34 |  | CA228FM6 | 220uF 25V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C101, 102 |  | CA107FM6 | 10uF ALUM ELECT CAPACITOR - 2+50% RADIAL |
| C103. 104 |  | CM105ML6 | 100nF 160V POLY CAPACITOR 20% RADIAL |
| C105 |  |  | AS C101 |
| C106 |  | CM225KL6 | 220nF 100V MET POLY CAPACITOR 20% RADIAL |
| C201/202 |  | CD102FL6 | 100pF 25V CERAMIC CAPACITOR 20% RADIAL |
| C203 |  | CM475KL6 | 470nF 100V MET POLY CAPACITOR 20% RADIAL |
| C204 |  | CM225KL6 | 220nF 100V MET POLY CAPACITOR 20% RADIAL |
| C205 |  | CM105ML6 | 100nF 160V MET POLY CAPACITOR 20% RADIAL |
| C206 |  | CA685JLO | 680nF 50V ALUM ELECT CAPACITOR 20% AXIAL |
| C207 |  | CA476HM6 | 4.7uF 35V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C208 |  | CM104TL6 | 10nF 630V MET POLY CAPACITOR 20% RADIAL |
| C210 |  | CP473GGO | 4700pF 30V POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C211 |  | CM105ML6 | 100nF 160V POLY CAPACITOR 20% RADIAL |
| C212 |  | CA228FM6 | 220uF 25V ALUM ELECT CAPACITOR - 2O+50% RADIAL |
| C213 |  | CM474NK6 | 47nF 250V MET POLY CAPACITOR 20% RADIAL |
| C214/215 |  | CM105NL6 | 100nF 250V MET POLY CAPACITOR 20% RADIAL |
| C216/217 |  | CD104YL6 | 10nF 2KV CERAMIC CAPACITOR 20% RADIAL |
| C218 |  | CL335KN6 | ! 0.33uF 250V POLYPROPYLENE CAP 10% RADIAL |
| C220 |  | CM104TL6 | 10uF 630V MET POLY CAPACITOR 20% RADIAL |
| C221 |  | CM475RL6 | 470nF 400V MET POLY CAPACITOR 20% RADIAL |
| C222 |  | CL753XJ6 | ! 7500pF 1500V POLYPROPYLENE CAP 5% RADIAL |
| C222 (ALL 1441, 1442 MODELS) |  |  |  |
|  | (1446/LF) |  |  |
|  | (1446/L12) |  |  |
|  | (1451/MQ3) | CL563YJ6 | 5600pF 2000V POLYPROPYLENE CAP 5% RADIAL |
| C222 | (1241/AS) | CL333XJ6 | 3n3 2000V POLYPROPYLENE CAP 5% RADIAL |
| C222 | (1241/AT) | CL473YJ6 | 4700pF 2000V POLYPROPYLENE CAP 5% RADIAL |
| C223 |  | CM684XK6 | 68nF 1 500V POLY CAPACITOR 20% RADIAL |
| C224 |  | CA109IN7 | 1000uF 40V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C225 |  | CD561RL6 | 56pF 400V CERAMIC CAPACITOR 20% RADIAL |
| C301 |  | CM225KL6 | 220nF 100V MET POLY CAPACITOR 20% RADIAL |
| C302 |  | CM105NK6 | 100nF 250V POLY CAPACITOR 20% RADIAL |

C

| 303 |  | CM155KK6 | 150nF 100V MET POLY CAPACITOR 20% RADIAL |
| --- | --- | --- | --- |
| C304 |  | CA108HM6 | 100uF 35V ALUM ELECT CAPACITOR - 20+50% RADIAL |
| C305/306 |  |  | AS C302 |
| C306 | (1241/AS) |  |  |
|  | (1241/AT) | CM225KL6 | 220nF 100V MET POLY CAPACITOR 20% RADIAL |
| C307 |  | CD122RL6 | 120pF 400V CERAMIC CAPACITOR 20% RADIAL |
| C308 |  | CA107FM6 | 10uF 25V ALUM ELECT CAP - 20+50% RADIAL |
| C310 |  | CM105NK6 | 100nF 250V POLY CAPACITOR 20% RADIAL |
| C311 |  | CA478HM6 | 470uF 35V ALUM ELECT CAP - 20+50% RADIAL |
| C312 |  | CA228FM6 | 220uF 25V ALUM ELECT CAP - 20+50% RADIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| C801 |  | CM105NL6 | 100nF 250V METAL POLYESTER CAPACITOR 20% RADIAL |
| C802/804/806 |  | CP271GGO | 27pF 30V POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C803/805/807 |  | CP561GG0 | 56pF 30V POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C808 |  | CD104YL6 | 10nF 2KV CERAMIC CAPACITOR 20% RADIAL |
| C810 |  | CM105NL6 | 100nF 250V METAL POLYESTER CAPACITOR 20% RADIAL |
| C811 | CD104YL6 10nF 1-2KV CERAMIC CAPACITOR 20% RADIAL |  |  |
| C901 |  | CM105NL6 | 100nF 250V METAL POLYESTER CAPACITOR 20% RADIAL |
| C902,904,906 |  | CP271GGO | 27pF POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C903,905,907 |  | CP122GG1 | 120pF POLYSTYRENE CAPACITOR 2.5% AXIAL |
| C910.911 |  | CD104YLS | 10nF 2KV CERAMIC CAPACITOR 20% RADIAL |
| C913 |  | CM105NL6 | 0.1uF 250V METAL POLYESTER CAPACITOR 20% |
| D1-4 |  | DP4007UMO | POWER RECTIFIER DIODE IN4007 MOTOROLA AXIAL |
| D5 |  | OP4005UMO | POWER RECTIFIER DIODE IN4005 MOTOROLA AXIAL |
| D6 |  | DF0157UEO | FAST RECOVERY DIODE BA157 MOTOROLA AXIAL |
| D7 |  | DF0159UEO | FAST RECOVERY DIODE BA159 MOTOROLA AXIAL |
| D8, 10-14 |  | DS4148UTO | SMALL SIGNAL DIODE IN4148 THOMPSON AXIAL |
| D11-13 |  | DS4148UTO | SMALL SIGNAL DIODE IN4148 THOMPSON AXIAL |
| D15,16 |  | DP4002UMO | POWER RECTIFIER DIODE 1N4002 MOTOROLA AXIAL |
| D17 |  | DF0159UEO | FAST RECOVERY DIODE BA159 MOTOROLA AXIAL |
| D18 |  | DZ3875CFCO | ZENER DIODE BZY88C7V5 400mW 5% AXIAL |
| D20 |  | D279331FCO | ZENER DIODE BZX79C33V 400mW 5% AXIAL |
| D21 |  | DP4002UMO | POWER RECTIFIER DIODE IN4002 MOTOROLA AXIAL |
| D22 |  | DF0159UEO | FAST RECOVERY DIODE BA159 MOTOROLA AXIAL |
| D23 |  | DF0818UMO | FAST RECOVERY DIODE MR818 MOTOROLA AXIAL |
| D24 |  | DF0157UEO | FAST RECOVERY DIODE BA157 MOTOROLA AXIAL |
| D25,116 |  | DP4002UMO | POWER RECTIFIER DIODE IN4002 MOTOROLA AXIAL |
| D101-107 |  | DS4148UTO | SMALL SIGNAL DIODE IN4148 THOMPSON AXIAL |
| D108,110,111 |  | DZ79560FCO | ZENER DIODE BZX79C5V6 400mW 5% AXIAL |
| D108,110,111 |  |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (2030/CS5) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS4) |  |  |
|  | (1439/MS) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) | DZ79430FCO | ZENER DIODE BZX79C4V3 400mW 5% AXIAL |
| D117 |  | DZ79121FCO | ZENER DIODE BZX79C12V 400mW 5% AXIAL |
| D118 |  |  | AS D108 |
| D201 |  | DF0157UEO | FAST RECOVERY DIODE BA157 MOTOROLA AXIAL |
| D301 |  | DS4148UTO | SMALL SIGNAL DIODE 1N4148 THOMPSON AXIAL |
| D302 |  | DP4005UMO | POWER RECTIFIER DIODE IN4005 MOTOROLA AXIAL |
| D801-803 |  | DS4148UTO | SMALL SIGNAL DIODE IN4148 THOMPSON AXIAL |
| D804 |  | DZ79750FC1 | ZENER DIODE BZX79C75 400mW 5% AXIAL |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| D804 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1442) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | DZS8750FCO | DIODE BZY88C7V5 |
| D805-808 |  | DS4148UTO | SMALL SIGNAL DIODE 1N4148 THOMPSON AXIAL |
| D901-906 |  | DS4148UTO | SMALL SIGNAL DIODE IN4148 THOMPSON AXIAL |
| D907 |  | DZ79750FC1 | ZENER DIODE BZX79C7V5 400mW 5% AXIAL |
| D907 | (1496/LI2U) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1456/LI2) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451/AS) | DZ88750FCO | ZENER DIODE BZY88C7V5 400mW 5% AXIAL |
| D908 F1/F2 |  | KA2001BAO | AS D805 ! FUSE 2AMP (20mm) TIME DELAY |

_____________TRIPLE STANDARD - PAL INTERFACE	_________

GENERAL

The 'Triple Standard - PAL Interface Assembly' is designed to interface between the Series 3 main chassis PCB assembly and the host system. It is designed to accept RS170 video signals (0.7v p-p video + 0.3v p-p mixed - Ve Sync into 75R). In wire-frame chassis format, output connections from interface to main chassis PCB are made via

hard-wired flexible leads and rnultiway plug-in connectors PL102 and PL103, located on the mam chassis.

On some models, an interface input wiring harness lead assembly is provided. This is supplied with one end only of the harness terminated in a 17-way connector, the opposite wire ends being unterminated. Circuit details and connections for this harness lead assembly are given in the accompanying diagram - ' 1 7-Wav Harness Connections' and Table of Options'.

DOUBLE STANDARD/TRIPLE STANDARD INPUTS

Depending on how the interface assembly is installed, input modes for either double or triple standards may be accommodated as follows:

1.         TTL and PAL Video Mode - Double Standard

2.         TTL, 1 volt 75 ohm and PAL Video Mode - Triple Standard

1.         Double Standard Input

When wired as indicated in the diagram - TTL' and 'PAL Video' Modes are accommodated. Either one of these two modes may be selected by a 'single pole change-over' switch as shown in the diagram.

2.         Triple Standard Input

Wiring details to accommodate Triple-Standard inputs and the various input options are given in the' 17-Way Harness Connections' Diagram and the accompanying Table of Options'. Suitable additional switching may be incorporated to accommodate the various options, depending on user require-ments.

OPERATIONAL NOTES

1.         PAL/Sync Input

When this input (PL1-pin 16) is used as the separate sync input of a 1v/75R R.G.B. + sync video source, the amplitude of the sync MUST BE between 0.2v p-p and 0.7v p-p, nominally 0.3v peak to peak. If the amplitude exceeds these values, an 'In-Line' B.N.C. attenuator must be used.

2.         Sync-On Green Option

Setting Plug Link TL1' on the PAL Interface PCB to 'Position 2', allows a 'sync-on-green' 1V/75R RGB Video Source to be used.

NB: Note that the 'Green' 1V/75R video input (PL1-pin 12) now also becomes the PAL input. See wiring diagram and Table of Options'.

BRIGHTNESS CONTROL LIMIT

If required - a resistor of 5K6 ohms 0.25W - may be incorporated in series with the 'brightness' control to limit the control's range of 'brightness' variation - as shown in the diagram.

AUDIO STAGE

On some versions of the interface, either single channel or twin channel audio amplifier stages may be incorporated, depending on model and specification.

Inputs and outputs for the audio stage(s) are marked as left-hand channel (L) and right-hand channel (R) and are terminated in 2-pin connectors. Normally, only a single channel audio stage is provided in most applications.

Audio input to the stage may be fed across a 47k 'gain control' pot, with the slider taken to the audio stages input.

T SERIES CABINET - 14 INCH MODEL

A foam polyurethane moulded cabinet incorporating a swivel base assembly T SERIES CABINET COMPONENT PARTS

| IDENT NO. | PART NO. | DESCRIPTION |
| --- | --- | --- |
| 1 | PC5047I02 | FACIA. CABINET BEZEL MOULDING |
| 2 | PC5046I01 | CABINET COVER MOULDING |
| 3 | PC5045IO1 | CABINET BASE MOULDING |
| 4 | PC5044I01 | CABINET PLINTH MOULDING |

M SERIES CABINET - 14 AND 20 INCH MODELS

This cabinet is an all metal, rugged construction. The 20 inch model was deleted from production in December 1983.

M SERIES CABINET COMPONENT PARTS

| IDENT NO. | PART NO. | DESCRIPTION |
| --- | --- | --- |
| 1 | MC0066I04 | FACIA. CABINET BEZEL |
| 2 | MC0021I05 | CABINET TOP/SIDE ASSEMBLY |
| 3 | MC0019I03 | CABINET BASE ASSEMBLY |
| 4 | MC0094I03 | CABINET BACK ASSEMBLY |

L SERIES CABINET - 14 INCH MODEL

This cabinet is constructed of high density structural foam. A modern compact design, available in different colours.

L SERIES CABINET COMPONENT PARTS

| IDENT NO. | PART NO. | DESCRIPTION |
| --- | --- | --- |
| 1 | PC0103I05 | FACIA. CABINET BEZEL |
| 2 | PC0104I02 | CABINET TOP/SIDE ASSEMBLY |
| 3 | PC0107I02 | CABINET BASE ASSEMBLY (MOULDED) |
| 4 | P00308I01 | CABINET BASE PANEL (MOULDED) |
| 5 | PC0105I01 | CABINET BACK ASSEMBLY |
| 6 | M00141I03 | INPUT AND CONTROLS BRACKET ASSEMBLY (METAL) |

D SERIES CABINET - 14 INCH MODEL

This cabin is injection moulded, and is one of the latest designs offered in the MICROVITEC CUB monitor range and is available in different colours.

D SERIES CABINET COMPONENT PARTS

| IDENT NO. | PART NO. | DESCRIPTION |
| --- | --- | --- |
| 1 | PC0124I01 | FACIA, CABINET BEZEL |
| 2 | PC0125I01 | CABINET |
| 3 | PC0126I01 | HINGED DOOR |
| 4 | M00221I04 | INPUT BRACKET ASSEMBLY (METAL) |

C SERIES CABINET - 20 INCH MODEL

This cabinet is an all metal, rugged construction, incorporating carrying handles fitted on each side.

C SERIES CABINET COMPONENT PARTS

| IDENT NO. | PART NO. | DESCRIPTION |
| --- | --- | --- |
| 1 | PC0136I03 | FACIA, CABINET BEZEL |
| 2 | MC0137I04 | CABINET TOP/SIDE ASSEMBLY |
| 2 | MC0150I03 | CABINET TOP/SIDE ASSEMBLY (INCL. SPEAKER) |
| 3 | M00235I02 | CABINET BASE ASSEMBLY |
| 4 | MC0139I01 | CABINET BACK ASSEMBLY |
| 5 | PC0140I01 | BLISTER. FITTED TO CABINET BACK |

| COMP REF |  |  |  |
| --- | --- | --- | --- |
|  | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| TR802,805,807 |  |  |  |
| 902,904,906 |  | QS0787UM1 | NPN SILICON TRANSISTOR BF787-5 MOTOROLA |
| TR802,805,807 |  |  |  |
| 902,904,906 | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1435/MS) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS}) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | QS0869UAO | NPN SILICON TRANSISTOR BF869-5 MOTOROLA |
| TR802,805 |  | QS0869UAO | TRANSISTOR BF869 |
| TR803,907 |  | QS4123UMO | NPN SILICON TRANSISTOR 2N4123 MOTOROLA |
| TR806 |  | QS0393WT1 | TRANSISTOR BF393 W1 |
| TR807 | (1441) |  |  |
|  | (1442) |  |  |
|  | (1241/AS) | QS0869UAO | TRANSISTOR BF869 |
| TR902,904,906 |  |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1496/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1456/LI2) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) (2032/AM) | QS0869UAO | NPN SILICON TRANSISTOR BF869-5 MOTOROLA |
| TRIPLER |  | ET001QA1 | TRIPLER BG1897 |
| TY1,2 |  | QY0103UMO | THYRISTOR BR 103 MOTOROLA AXIAL |
| PCB |  | BC0029106 | MAIN PCB ASSEMBLY SERIES 3 (6) |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| TR2 |  | QT002AXO | NPN DARLINGTON TRANSISTOR TIPL 753A (R3213) |
|  | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431 /LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1456/LI2) |  |  |
|  | (2032/AM) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | QP0753UXO | NPN TRANSISTOR TIPL753A (R321 3) |
| HEATSINK |  | A00007102 | LCCD HEAT SINK (SMPSU) ASSEMBLY |
| TR2 | HEATSINK | M00205I01 | HEATSINK REDPOINT TV3 SPECIAL |
| TR16 |  | QS0500UMO | NPN SILICON POWER TRANSISTOR MOTOROLA BU500 |
|  |  | OR |  |
|  |  | QS0500VXO | NPN SILICON POWER TRANSISTOR TEXAS BU500 |
| TR16 |  |  |  |
| HEAT SINK |  | A00008102 | LCCD HEAT SINK (LOPT) ASSEMBLY |
| TR101 |  | QS0337UTO | NPN SILICON TRANSISTOR BC337-5 MOTOROLA |
| TR102 |  | QS4123UMO | NPN SILICON TRANSISTOR 2N4123 MOTOROLA |
| TR103-105 |  | QS4125UM5 | PNP SILICON TRANSISTOR 2N4125 MOTOROLA |
| TR106 |  | QS0337UTO | NPN SILICON TRANSISTOR BC337-5 MOTOROLA |
| TR201 |  | QS0460UMO | NPN SILICON TRANSISTOR BF460-5 MOTOROLA |
| TR201 |  |  |  |
| HEATSINK |  | M00305I01 | HEATSINK REDPOINT TV3 SPECIAL |
| TR202 |  | QS0500UMO | NPN TRANSISTOR SIL BU500 |
| TR202 |  |  |  |
| HEATSINK |  | M00205I01 | HEATSINK REDPOINT TV3 SPECIAL |
| TR301 |  |  | AS TR106 |
| TR801,804,806 |  |  |  |
| 901,903,905 |  | QS0392UMO | NPN SILICON TRANSISTOR BF392 MOTOROLA |
| TR801-804 |  |  |  |
|  | (1441,1442) |  |  |
|  | (1449/AS) | QS0393WT1 | TRANSISTOR BF393.W1 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| TL201 | (2031/CS5) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AT) | KL4838Z02 | LINK TEST 10/4838 PRESSAC |
| TL201A/B |  |  |  |
|  | (2031/CS5) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1441/MS4) |  |  |
|  | (1449/AS) | KM3070Y01 | PIN TEST 10/3070 PRESSAC |
| TL202-2 |  | WL2210TU1 | WIRE LINK 5mm + 10mm + 5mm |
| TL901 |  | KM3070Y01 | PIN TEST 10/3070 PRESSAC |
| TL901 |  | KL4838Z02 | LINK TEST 10/4838 PRESSAC |
| TL901 |  | KM0006Y01 | PIN TURRET 10/006 PRESSAC |
| TL901A/B |  |  |  |
|  | (1435/MS) | KM3070Y01 | PIN TEST 10/3070 PRESSAC |
| TL901A/B |  |  |  |
|  | (1435/MS) | KL4338Z02 | LINK TEST 10/4838 PRESSAC |
| TR1 |  | QS0307UM5 | PNP SILICON TRANSISTOR BC307-5 MOTOROLA |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| TL102A/B, |  |  |  |
| 201A/B |  |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | KM3070Y01 | PIN TEST 10/3070 PRESSAC |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| TL101.103-1C6 |  |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AT) | KS7859I02 | SOCKET-SHORTING 7859-02 |
| TL102 |  |  |  |
|  | (1486/LI2U) |  |  |
|  | (1435/MS) |  |  |
|  | (1446/LI2) | KL4838Z02 | TEST LINK 10/4838 PRESSAC |

| COMP REF | MODEL NO. | PART COMPONENT NO. DESCRIPTION |  |
| --- | --- | --- | --- |
| TL101-106 |  |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
| (1431,AS) |  |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | KP0024A03 | 3-WAY PLUG 20/3423 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| TH2 | (1496/LI2U) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1439/AS) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AT) | RT003NNO | THERMISTOR 15 OHMS 30% K231 |

I

| COMP REF | MODEL MO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| T1 |  |  | LINKS 2 x 14mm |
| T1A/1B |  | LM0001UA1 | CHOKE (OREGA) 88545-00 |
| T2 |  | TI0004I06 | TRANSFORMER SWITCH MODE ISOLATING PC5287 OR |
|  |  |  | PC5307 |
| T2 | (1496/LI2U) |  |  |
|  | (1486/LI2U) | TL002SU0 | TRANSFORMER SWITCH MODE ISOLATING PC3960 |
| T201 |  | TI0009I01 | TRANSFORMER LINE DRIVER PC4163 |
| T202 |  | TI0003I01 | ! TRANSDUCTOR EAST-WEST PC3396 |
| T203 |  | TI0007I01 | TRANSFORMER LINE OUTPUT 14" PC3755 |
| T203 | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) | TI0016I01 | TRANSFORMER LINE OUTPUT 20" |
| TH1 | (1496/LI2U) |  |  |
|  | (1486/LI2U) | RT002QL0 | THERMISTOR PTH45LC09-BG8ROH140 |
| TH1 | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AT) | RT005QN0 | THERMISTOR 263100P2332T333 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| PL3 | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS 4) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | KP0300D05 | 5-2 PIN INLINE PLUG MTG SHROUDED LOCKABLE, |
|  |  |  | PRESSAC 320/3765 |
| PL101 |  | KP0026A10 | 10 PIN INLINE PLUG PCB MTG SHROUDED PRESSAC |
|  |  |  | 20/3450 |
| PL102 |  | KP0025A05 | 5-2 WAY INLINE PLUG PCB MTG 20/3345 |
| PL103 |  | KP0025A10 | 10 PIN INLINE PLUG PCB MTG UNSHROUDED PRESSAC |
|  |  |  | 20/3430 |
| PL103 |  |  |  |
|  | (2030/CS5) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1439/AS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1449/AS) | KP0026A10 | 10 PIN INLINE PLUG PCB MTG SHROUDED PRESSAC |
|  |  |  | 20/3450 |
| PL201 |  | KP0222D08 | 8 PIN INLINE PLUG PCB MTG UNSHROUDED PRESSAC |
|  |  | OR | 220/1138 |
|  |  | KP0300D08 | 8 PIN INLINE PLUG PCB MTG UNSHROUDED LOCKABLE, |
|  |  |  | PRESSAC 320/3768 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| LK 1-5,7,8 |  |  |  |
| 10-18,20-22 |  | WL2214TU1 | WIRE LINK 5mm x 14mm x 5mm |
| LK23 | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) | WL2214TU1 | WIRE LINK 5mm x 14mm x 5mm |
| PL1 |  | KP0220D06 | PIN INLINE PLUG PCB MTG SHROUDED PRESSAC |
|  |  |  | 20/1546 |
| PL1 | (1496/L12U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/L12) |  |  |
|  | (1241/AT) | KP0300D06 | 6 PIN INLINE PLUG PCB MTG SHROUDED LOCKABLE, |
|  |  |  | PRESSAC 320/3766 |
| PL2 |  | KP0025A03 | PLUG 3 WAY 20/3443 |
| PL3 |  | KP0220D05 | 5-2 PIN INLINE PLUG MTG SHROUDED PRESSAC 220/2145- |
|  |  |  | 2,4 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| L925 |  | LW154UA5 | 5uH (IN SERIES WITH R925) |
|  | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431 /LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1456/LI2) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) | LW154SK2 | CHOKE 15uH B78108-T1153-K |
| L926 |  | LW154UA5 | 15uH (IN SERIES WITH R926) |
| L926 | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1456/LI2} |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) LW154SK2 CHOKE 15uH B78108-T1153-K |  |  |

C

| OMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| L901 | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1456/LI2) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) | LW154SK2 | CHOCK WIRE ENDED 15uH B78108-T1153-K |
| L924 |  | LW154UA5 | 15uH (IN SERIES WITH R924) |
|  | (1496/LI2U) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/M23) |  |  |
|  | (1431/M24) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS4) |  |  |
|  | (1451AP/MS4 |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MQ3) |  |  |
|  | (1456/LI2) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) | LW154SK2 | CHOKE 15uH B78108-T1153-K |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| L2 | (1496/LI2U) |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1486/LI2U) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/DS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | LW104UA7 | CHOKE WIRE ENDED 10uH PC5640 |
| L201 |  | LW154SK2 | CHOKE WIRE ENDED 15uH B78108-T1153-K |
| L202 |  | LV001UA5 | COIL WIDTH PC3398 |
| L203 |  | LN002UA5 | COIL LINE LINEARITY PC5580 |
| L204 |  | LW104UA7 | CHOKE WIRE ENDED 10uH PC5640 |
| L801-804 | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1442) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | LW154SK2 | CHOKE 15uH B78108-T1153-K |
| L801 |  | LW154VA5 | CHOKE WIRE ENDED 15uH PC3391 |
| L802 |  | LW104VA4 | CHOKE WIRE ENDED 10uH PC2677 |
| L803/804 |  | LW104VA4 | CHOKE WIRE ENDED 10uH PC2677 |
| L901 |  | LW154UA5 | CHOKE WIRE ENDED 15uH PC3391 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| FH3A/B | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | KS0001Y01 | FUSE CLIP-PCB MTG (5mm) |
| HARNESS |  | A00584I01 | TB -NO. 1 (4 LEADS) BR, OR, W, Y. |
| HARNESS |  | A00585I01 | TB - NO. 2 (8 LEADS) |
| HARNESS | (2031/CS5) |  |  |
|  | (1449/AS) | A00815I01 | TB - NO. 1 (4 LEADS) BR, OR, W, Y. |
| HARNESS | 2031/CS5) | A00816I01 | TB - NO. 2 (8 LEADS) |
| HARNESS | (1441/AL) | A00104I01 | TB - NO. 1 (4 LEADS) |
| HARNESS | (1441/AL) | A00179I01 | TB - NO. 2 (8 LEADS) |
| IC1 |  | IV7812MX3 | VOLTAGE REGULATOR 1C 78M12 |
| IC1 |  |  |  |
| (HEATSINK) |  | HH0003HAO | HEATSINK 1 STAVER TYPE V6-2L |
| IC101 |  | IT74136MM2 | OR/TTL LOGIC SN74LS136N |
| IC102 |  | IV7805LXO | VOLTAGE REGULATOR IC "1" 78L05 |
| IC201 |  | IL1180PS2 | LINEAR Bl-POLAR IC TDA 1180P PLASTIC DIL S.G.S. |
| IC301 |  | IL1170SS2 | LINEAR Bl-POLAR IC TDA 1170S PLASTIC DIL S.G.S. |
| IC301 |  |  |  |
| (HEATSINK) |  | HHC001HCO | HEATSINK STAVER V8-800 |
| L2 |  | LW473UA6 | CHOKE WIRE ENDED 10uH PC3774 |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| FH1A/B,FH2A/B |  |  |  |
|  | (2030/CS5) |  |  |
|  | (2031/CS5) |  |  |
|  | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1449/AS) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1241/AT) | KS0003Y01 | FUSEHOLDER L222/K |
| FH3A/B | (1496/LI2U) |  |  |
|  | (1486/LI2U) | KS0006Y01 | FUSE CLIP-PCB MTG (6.3mm) |

| COMP REF | MODEL NO. | PART NO. | COMPONENT DESCRIPTION |
| --- | --- | --- | --- |
| F2 | (2031/AX) |  |  |
|  | (2032/AM) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431ap/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451AP/DS) |  |  |
|  | (1451/DS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AT) | KA2001BAO | ! FUSE 2AMP (20mm) TIME DELAY |
| F3 | (2031/AX) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431AP/MS4) |  |  |
|  | (1431AP/DS) |  |  |
|  | (1431/AS) |  |  |
|  | (1432/AS) |  |  |
|  | (1431/BS) |  |  |
|  | (1431/LS1) |  |  |
|  | (1431/MZ3) |  |  |
|  | (1431/MZ4) |  |  |
|  | (1431/MB4) |  |  |
|  | (1431/MR4) |  |  |
|  | (1431/MS4) |  |  |
|  | (1431/DS) |  |  |
|  | (1435/MS) |  |  |
|  | (1436/LS1) |  |  |
|  | (1436/MS4) |  |  |
|  | (1439/AS) |  |  |
|  | (1439/MS) |  |  |
|  | (1441/AL) |  |  |
|  | (1441/AS) |  |  |
|  | (1441/MS) |  |  |
|  | (1441/MS4) |  |  |
|  | (1451/AS) |  |  |
|  | (1451/MS) |  |  |
|  | (1451AP/MS4) |  |  |
|  | (1451/DS) |  |  |
|  | (1446/LF) |  |  |
|  | (1449/AS) |  |  |
|  | (1241/AS) |  |  |
|  | (1446/LF) |  |  |
|  | (1446/LI2) |  |  |
|  | (1456/LI2) |  |  |
|  | (1241/AS) | KA1001BQO |  |
| FH1 A/B | A/B | KS0001Y01 |  |

TYPICAL AUDIO SPECIFICATIONS

Typical specifications for the audio amplifier stage are as follows:

| Input impedance: |  | 47 k nominal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Input sensitivity: |  | 100 mV typical |  |  |  |  |
| Output: |  | 2 watts max @ 1kHz into 16 ohms |  |  |  |  |
| TABLE OF OPTIONS |  |  |  |  |  |  |
| Connector |  |  | Available Standards |  |  |  |
| Pin No. | Wire Colour Coding |  | TTL | 1V/75R |  | PAL |
| 10 | Brown |  | O/C | 0V (S/C) |  | 0V (S/C) |
| 15 | Mauve |  | • | > 0.7V < 12V (O/C) |  | 0V (S/C) |
| 17 | Pink |  | 0V (S/C) | > 10.5V < 12V (O/C) |  | > 10.5V < 12V (O/C) |

NOTES:

-    Not Critical (May be 0V to 12V or O/C)

0/C   =    Open Circuit (No connection)

S/C   =    Short Circuit (< 10 R) to 0V (Ground)

PARTS LIST

| Circuit Reference | Component Reference | Component Description |
| --- | --- | --- |
| RESISTORS |  |  |
| R5, 20, 21, 23, 25, 27, 28 | RF104DJO | RESISTOR C/F 10K 1/4W 5% |
| R6, 8 | RF562DJO | RESISTOR C/F 560R 1/4W 5% |
| R7, 30 | RF473DJO | RESISTOR C/F 4K7 1/4W 5% |
| R9 | RF156DJO | RESISTOR C/F 1M5 1/4W 5% |
| R10 | RF183DJO | RESISTOR C/F 1K8 1/4W 5% |
| R11, 12 | RF392DJO | RESISTOR C/F 390R 1/4W 5% |
| R13, 18 | RF225DJO | RESISTOR C/F 220K 1/4W 5% |
| R14, 15 | RF123DJO | RESISTOR C/F 1K2 1/4W 5% |
| R16 | RF272DJO | RESISTOR C/F 270R 1/4W 5% |
| R17 | RF273DJO | RESISTOR C/F 2K7 1/4W 5% |
| R19 | RF184DJO | RESISTOR C/F 18K 1/4W 5% |
| R22, 24, 26 | RF103DJO | RESISTOR C/F 1K0 1/4W 5% |
| R31 | RL101DJO | RESISTOR M/FUS, 10R 1/4W 5% |
| R32 | RF392DJO | RESISTOR C/F 390R 1/4W 5% |
| R33 | RF152DJO | RESISTOR C/F 150R 1/4W 5% |
| R40 | RF103DJO | RESISTOR C/F 1KO 1/4W 5% |
| R101, 201 | RF104DJO | RESISTOR C/F 10K 1/4W 5% |
| R102, 202 | RF222DJO | RESISTOR C/F 220R 1/4W 5% |
| R103, 203 | RF100DJO | RESISTOR C/F 1R0 1/4W 5% |
| VR1 | RQ102AL2 | POT PRESET 100R 0.1W H7 |
| CAPACITORS |  |  |
| C1,2 | CA337EN6 | CAPACITOR ALUM/ELEC 33uF 16V |
| C3 | CK331JJO | CAPACITOR CERAMIC/T 33pF 50V |
| C4 | CK121JKO | CAPACITOR CERAMIC/T 12pF 50V |
| C5 | CK681JJO | CAPACITOR CERAMIC/T 68pF 50V |
| C6, 8-15 | CK104FLO | CAPACITOR CERAMIC/T 10nF 25V 20% |
| C7 | CM225KK6 | CAPACITOR MET/T 0.22uF 100V |
| C16, 17, 18, 19, 20, 27 | CM105NL6 | CAPACITOR MET/T 0.1uF 250V |
| C21 | CK222JKO | CAPACITOR CERAMIC/T 220pF 50V |
| C22 | CM105NL6 | CAPACITOR MET/P 0.1uF 250V |
| C23 | CK103JKO | CAPACITOR CERAMIC/T 1nF 50V |
| C24 | CK151JKO | CAPACITOR CER/T 15pF 50V AXIAL |
| C25, 26 | CM4758K6 | CAPACITOR MET/P 470nF 63V |
| C30 | CA107JL7 | CAPACITOR A/ELEC 10uF 50V 20% RAD PR |
| C31 | CA105NL6 | CAPACITOR CERAMIC/T 100nF |
| C101, 102, 201, 202 | CA2268M7 | CAPACITOR A/ELEC 2.2uF 63V PREF |
| C103, 203 | CA107JL7 | CAPACITOR A/ELEC 10uF 50V 20% RAD PR |
| C104, 204 | CM225KL6 | CAPACITOR MET/P 0.22uF 100V |
| C105, 205 | CA109HM6 | CAPACITOR ALUM/ELEC 1000uF 35V |
| C106, 206 | CM105NL6 | CAPACITOR MET/P 0.1uF 250V |
| C107, 207 | CA478FM7 | CAPACITOR ALUM/ELEC 470uF 25V |

| Circuit Reference | Component Reference | Component Description |
| --- | --- | --- |
| DIODES |  |  |
| D1, 2, 3 | DZ73560FRO | DIODE ZENER BZX79B5V6 2% |
| D4, 5 | DS4143UTO | DIODE 1N 4148 THOMSON |
| INTEGRATED CIRCUITS |  |  |
| IC1 | IL3301UM3 | CIRCUIT INT. TDA3301 SELECTED |
| IC2 | IG4551UM2 | CIRCUIT INT. 4551 |
| IC101, IC201 | IL19080S2 | CIRCUIT INT. TDA 1908 |
| CHOKES & DELAY LINES |  |  |
| L1 | LW474SK2 | CHOKE 47uH |
| L2 | LW105SK2 | CHOKE 100uH |
| L3 | LW104SK2 | CHOKE 10uH |
| L4, 5 | LV001TA3 | CHOKE KAN K2819XM |
| L11 | LW154SK2 | CHOKE 15uH B78108-T1153-K |
| DL1 | ED0001P01 | DELAY LINE DL470 |
| DL2 | ED0002P01 | DELAY LINE DL711 |
| CONNECTORS |  |  |
| PL1 | KP0026A17 | PLUG 17-WAY 20/3457 |
| PL2 | KP0026A16 | PLUG 16-WAY 20/3456 |
| PL102 | KP0025A05 | PLUG 5-WAY 20/3445 |
| PL103 | KP0025A03 | PLUG 3-WAY 20/3443 |
| PL104 | KP0025A04 | PLUG 4-WAY 20/3444 |
| TL1 | KP0024A03 | PLUG 3-WAY 20/3423 |
| MISCELLANEOUS |  |  |
| FIT TL1 | KL9005Z02 | LINK TEST MOLEX 90059-0009 P/0 |
| X1 | XC0014UU6 | CRYSTAL IQD TYPE 'P' A124D |
| 2 TRACK CUTS | BC0111I02 | PCB PAL INTERFACE |
| LK1, 2, 3 | WL2214TU1 | WIRE LINK 5mm x 14mm x 5mm |
| LK6 7, 8 | WL2212TU1 | WIRE LINK 5mm x 12mm x 5mm |
| LK9, 10 | WL2214TU1 | WIRE LINK 5mm x 14mm x 5mm |
| LK12, 13 | WL2212TU1 | WIRE LINK 5mm x 12mm x 5mm |
|  | A01647101 | I/FACE ASSEMBLY TPL/STD - MC REV 3 |

SPECTRUM INTERFACE CARD - DESCRIPTION/OPERATION

1.       LUMINANCE CHANNEL (Y)

A.       The (-Y) signal contains:

(1)	Video information

(2)	line sync pulses

(3)	frame sync pulse

3.       The incoming Y signal is attenuated by the potential divider, R16, R17 the resultant signal is then amplified by TR6.

C.       Transistors TR7, TR8 and TR9 have the following functions:-

(1)     TR7 - Separates the sync pulses

(2)     TR8 - Inverts the sync pulses

(3)     TR9 - Buffers the sync pulses

R28 and R29 in the emitter of TR9, splits the voltage swing to produce a 'SYNC TTL' output.

D.      The rising edge of the sync output signal is used by C6, R3Q and TR24 to produce a pulse which has the same duration and timing as the colour burst gating pulse on the +/- (R-Y) signal.

E.      The luminance signal on the collector of TR6, is clamped to the black level by C10 and TR25.

F.       The black level is set by resistor/diode potential divider formed by VR1, (R36. R37, R65 and 2 IN4148 diodes).

2.      COLOUR DIFFERENCE CHANNELS A.      The -(B-Y) Channel

(1)     The -(B-Y) signal is attenuated by a potential divider network R32 and R34. TR10 inverts and amplifies to produce + (B-Y). TR11 and C9 use the sync pulses to clamp the + (B-Y) signal to

the   black level.

(2)     TR21 merges + (B-Y) clamped signal to +Y clamped signal to produce the BLUE output.

(3)     TR22, is an electronic switch that has its threshold set by R56 and R57.

(4)     The gain of TR22 is set by R58 and R59 to produce TTL levels. TR23 emitter follows the TTL signal which produces a buffered output.

B.      The +/- (R-Y) Channel

This channel is a phase alternating line (PAL) signal, which requires alternate lines, inverted to produce + (R-Y).

(1)     The incoming +/- (R-Y) signal is A.C. coupled by C17, C1 and TR1 thus forming a unity gain, phase splitter.

(2)     The non inverted output of TR1 drives an electronic switch (TR3). The inverted output of TR1 is buffered by TR2.

(3)    The (R-Y) switch is enabled during the colour burst, gating pulse.

(4)     A set/re-set flip-flop switch formed by IC 1C and IC 1D works in the following :-

(a)    SET - If next line of video is POSITIVE

(b)    RESET - If next line of video is NEGATIVE

(5)     The outputs of 1C1D enable bi-directional switches which will either:

(a)    Connect +(R-Y) through C1

(b)    Connect inverted -(R-Y) through C2.

Thus producing +(R-Y). TR4 and TR5 clamp the +(R-Y) to the black level.

(6)     The + (R-Y) clamped signal is added to the +Y clamped signal by TR13 to produce +R signal.

(7)     TR14 has its threshold set by R43 and R42. the gain is set by R44 and R45, in order to produce TTL levels.

(8)     TR15 emitter follows the TTL signal thus producing a buffered signal output.

C.      THE (G-Y) OUTPUT

(1)     This signal is generated by mixing the following:-

(a)    The (B-Y) clamped signal. TR16 performs this mixing function

(b)    The (R-Y) clamped signal, TR16 performs this mixing function

(2)     The proportions of (B-Y) and (R-Y) clamped signals which are added together are set by R54 and R40.

(3)     G OUTPUTS

(a)    This signal is produced by the addition of the following signals:-(i)    (G-Y) clamped signal. TR17 performs this function. (ii)    +Y clamped signal, TR17 performs this signal function.

(4)     TR18 has its threshold set by R48 and R49. the gam is thus set by, R50 and R51 in order to

produce suitable TTL levels. (5)     TR19 emitter follows the TTL signal and produces a buffered output.

__________________'PROM' INTERFACE PANEL_________________

GENERAL

1.      The 'Programmable-read-only-memory' (PROM) interface panel is used with certain models in the 'SERIES-3' range of colour monitors. Two different versions of the panel assembly may be used -depending on model and intended country of operation.

2.      Basically, the two versions are very similar.

(1)     Versions incorporating R.F.I. filtering networks (L/C components L1   to L3 & C4 to C6), are employed with certain models intended for use in countries other than the UK.

(2)     Versions without R.F.I. filtering are currently used for models operating in the U.K.

3.      Interface connections:

(1)     When installed, this panel assembly interfaces between input connector PL101, located on 'SERIES-3' main chassis PCB and the monitor's 'user' 7-pin Din input socket.

NOTE: The 'customer' contrast control VR111, shown on the 'SERIES-3' main chassis circuit diagram, is not fitted as shown when the PROM interface is used. In this case, the 'customer' contrast control pot is connected to 'PL2', located on the PROM interface panel.

(2)     The 'PROM-INTERFACE CONNECTION DIAGRAM' provides wiring interconnection details. CIRCUIT DIAGRAM

1.      The circuit diagram for the 'PROM INTERFACE PANEL' provides circuit details of the panel assembly. Component values are given in the accompanying LIST of COMPONENTS.

CIRCUIT DESCRIPTION

1. The purpose of the 'PROM' interface panel is to convert 4-Bit digital video signals, normally referred to as 'R.G.B. & Intensity', into Linear R.G.B. signals.

2.     Colour Combinations

4

(1)    There are 16 possible colours which can be displayed (2 combinations).

(2)     IC2, a fusible link PROM, provides sixteen 8-bit outputs corresponding to one of the 16 addresses selected by the R.G.B. and intensity signals.

These outputs use 3-bits for RED and GREEN, and 2-bits for BLUE video, hence; there are 8 levels of Red and Green and 4 levels of Blue available.

(3)     Resistors R3 to R10 and resistors R11 to R18 form potential dividers respectively between base and collector resistors of TR101, TR102, TR103, located on the 'SERIES-3' main PCB. Resistors values are selected to 'weight' respective PROM outputs, to 'least significant' and 'most significant' bits.

NOTE: When the 'PROM interface Panel' is installed, the moveable links PL103, shown in the 'SERIES-3' main circuit diagram, should be fitted into linked POSITION (1).

3.     +5 Volt Supply

(1)    IC1, C2 and C3 provides a regulated + 5V supply for IC2.

4.      Protection & Termination

(1)     Diodes D1 to D5 provide flash-over protection for IC2 and for computer signal 'in' sources.

(2)     Reistors R21 to R26 may be fitted on some models to terminate incoming digital signals.

5.      Contrast Control

(1) Transfer TR1, R1 and an external pot connected to PL2 provide for 'customer' control of contrast on some models.

(2)    TR1 forms an emitter follower, the voltage at the emitter determining the video output amplitude on 'contrast'.

_________LIST OF COMPONENTS - PROM INTERFACE PANEL____________

GENERAL

1.      Components used in the Prom Interface Panel Assemblies, together with possible component variations

are listed following;

2.       Details of possible circuit variations, used with different versions of the panel, are shown in the

accompanying Prom Interface Circuit Diagram.

RESISTORS

1.      Most of the resistors employed in the construction are standard carbon film types of 0.25 watt rating.

±5% tolerance.

LIST OF COMPONENTS - PROM INTERFACE

| Component Reference | Part No. | Component Description |
| --- | --- | --- |
| RESISTORS |  |  |
| R1 | RF683DJO | RESISTOR C/F 6K8 0.25W 5% |
| R2 | RF391GJO | RESISTOR C/F 39R 0.5W |
| R3,6,11,14,20,21,22,23,24,25,26 | RF222DJO RF472DJO | RESISTOR C/F 220R 0.25W 5% RESISTOR C/F 470R 0.25W 5% |
| R4,7,10,12 |  |  |
| R15,18 | RF103DJO | RESISTOR C/F 470R 0.25W 5% |
| R5,8,13 | RF103DJO | RESISTOR C/F 1K0 0.25W 5% |
| R16 | RF683DJO | RESISTOR C/F 1K0 0.25W 5% |
| R9,17 | RF102DJO | RESISTOR C/F 100R 0.25W 5% |
| R19 |  | RESISTOR C/F 2K2 0.25W 5% |
| NOTE FOR APPLE MONITORS ONLY; - RESISTOR C/F R9,R17 BECOMES 220R 0.25W 5% |  |  |
| CAPACITORS |  |  |
| C1 | CA2268M6 | CAPACITOR ALUM/ELEC 2.2UF 63V |
| C2,3 | CM105ML6 | CAPACITOR MET/P 0.1 UF 160V |
| C4,5,6 | CK151JKO | CAPACITOR CER/T 15PF 50V |
| DIODES, TRANSISTORS, INTEGRATED CIRCUITS |  |  |
| TR1 | QS0337UTO | TRANSISTOR BC337-5 |
| IC1 | IV7805LXO | CIRCUIT INT. 78L05 |
| IC2 | IR18030AX2 | PROM TBP 18SA030N (PROGRAMMED INDIVIDUALLY FOR SPECIFIC MODEL) |
| D1,2-5 | DS4148UTO | DIODE IN4148 THOMPSON |
| CONNECTORS |  |  |
| PL1,3 | KP0026A10 | PLUG 10 PIN 20/3450 PRES |
| PL2 | KP0025A05 | PLUG 5 WAY 20/3445 |
| INDUCTORS |  |  |
| L1,2,3 | LW104SK2 | CHOKE 10 UH |

6.      Sync Signals

(1)    Sync signals are not required by the PROM circuitry and are passed directly through the interface panel.

7.      R.F.I. Filtering

(1)     On models requiring R.F.I. filtering, L/C networks formed by L1/C4, L2/C5 and L3/C6 are fitted on R. G & B outputs.

AMENDMENTS

AFFECTS

All metal cabinet monitors. DESCRIPTION

Modifications to bring cabinet back vvire routing, dressing and securing in line with current production practice.

IMPLEMENTATION

These modifications only apply whenever a monitor is returned for service.

MODIFICATIONS

With the cabinet top removed check the following:

Signal Wires On Cabinet Back Components

(DIN sockets. BNC sockets, mode switches, brightness/contrast controls etc).

a)       It is now the practice to fit a 'tyrap' fastener onto the signal wires on each component. (See diagrams). A 'tyrap' should be fitted to any component which is found not to have one.

Note: The signal wires are already mechanically twisted onto each pin and then soldered : this modification provides additional wire attachment.

b)      If it is possible to move any of these signal wires away from its normal position to touch the small exposed section of the terminals on the main switch then sufficient 'tyraps' should be fitted to these wires to hold them in position.

Note: This does not apply to wires which are already contained within a secondary sleeve. Single Insulated Mains Wires On Mains Switch

This only applies to the short sections of mains cable from which the outer sheath has been removed to allow the individual wires to be attached and soldered to the mains switch.

If it is possible to move any of these short sections of wire away from its normal position to touch a pin on one of the cabinet back components then sufficient 'tyraps' should be fitted to the mains wires to hold them in position. (See diagram).